Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title:	INDUSTRIAL ROBOTICS 								      Time: 3 hours

Subject Code:	09ME248 / 12ME245 / PE236			               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Which type of drive system used for pick and place robot?

2.	Which of the configuration would be most suitable for inserting a peg into a hole?

3.	What are the types of hydraulic actuators?

4.	Name the three degrees of freedom of robot’s wrist assembly?

5.	Mention any two contact type sensors.

6.	What is thresholding?

7.	What are robot programming methods?

8.	What is VAL?

9.	List the welding operations which can be performed by robots.

10.	Name the robot used for assembly operation.

PART – B (5 x 3 = 15 MARKS)

11.	Define the terms: work envelope and payload.

12.	What is reverse kinematics?

13.	What is proximity sensor?

14.	What is lead through programming?

15.	What is the significance of tele-operated robot?

PART – C (5 x 15 = 75 MARKS)

16.	Describe the basic motions of robot assembly with a neat sketch.

(OR)

17.	What are the drive systems used in robots?  Explain.

18.	Discuss the types of grippers used in robots for handling parts.

(OR)

19.	Determine a transformation matrix that represents a rotation through an angle ‘’ about OX axis, followed by a translation of ‘p’ units along OZ axis, followed by a rotation of ‘’ about the OY axis.

20.	Describe the non contact type sensors used in robots.

(OR)

21.	Describe how the image processing is carried out in machine vision system.

22.	What is AI? Explain the applications of AI in robots.

(OR)

23.	Four holes are to be made on a circular workpiece of diameter 80 mm.  These four holes are located in the pitch circle diameter of 40 mm.  Write a program to bring the end effector, holding a drill of diameter 4 mm to each location of the hole in a sequential order 1, 2, 3 and 4.

[P.T.O]

24.	Discuss the types of robots used for hostile and remote environment.

(OR)

25.	Describe the robot cell for loading and unloading a die casting machine, a trim press and a quench tank all working together.   Parts are fed to the chute for onward transmission to the part presenter from which the robot picks up the parts.