Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	ADVANCED PROPULSION SYSTEMS				      	     Time : 3 hours

Subject Code:	09AE211										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define Profile loss.

2.	Draw the velocity diagram for an Impulse turbine.

3.	During supercritical operation of the inlet the shock is formed _________.

4.	What is the operating range of a ramjet engine?

5.	What is the operating principle of a rocket engine?

6.	Why C-D nozzles are used in a rocket engine?

7.	What are the important hardware components of solid rocket?

8.	Write down the limitation of hybrid rockets.

9.	What is a solar sail?

10.	What are the different types of nuclear rocket engine?

PART – B  (5 x 3 = 15 MARKS)

11.	What are the various assumption made in practical cycle analysis in a gas turbine?

12.	How the scramjet engine differs from ramjet engine?

13.	Explain under expanded and over expanded nozzles.

14.	What are the advantages of liquid rocket over solid rocket?

15.	Draw the different parts of nuclear rocket engine.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	What are the different methods of blade cooling?    								    (7)

b.	Explain about matching of turbine and compressor.                                               		    (8)

(OR)

17.	Gas at 7 bar and 300 °C expands to 3 bar in an impulse turbine stage. The nozzle angle is 	70° with reference to the exit direction. The rotor blades have equal inlet and outlet angles and the stage operates with the optimum blade speed ratio. Assuming that the isentropic efficiency of the nozzles is 0.9 and that the velocity at entry to the stage is negligible, deduce the blade angle used and the mass flow required for this stage to produce 75 KW. Take Cp = 1.15 KJ/kg K.

18.	a.	What is the purpose of diffuser in the inlet? Explain.								    (5)

b.	What is the purpose of an aircraft gas turbine inlet and nozzle?						  (10)

(OR)

19.	a.	Describe the working of a ramjet engine. 									  	  (10)

b.	Depict the various thermodynamic processes occurring in it on h-s diagram.   		    (5)

20.	With the aid of neat sketches explain various techniques for thrust vector control.

(OR)

[P.T.O]

21.	A rocket projectile has the following characteristics

Initial mass                                                300kg

Mass after rocket operation                       180 kg

Payload, non propulsive structure etc       150 kg

Rocket operation duration                         5 sec

Average specific impulse of propellant     250 sec

Determine the vehicle’s mass ratio, Propellant mass fraction, propellant flow rate, thrust, thrust –to – weight ratio, acceleration of vehicle, effective exhaust velocity, total impulse and impulse to weight ratio.

22.	What are the important factors that influence the burning rate of a solid propellant? Explain them with appropriate sketches.

(OR)

23.	a.	What are the important factors in selecting a liquid propellant?			    			    (5)

b.	Explain the working of liquid propellant rocket engine with a gas pressure feed 	system.																		  			  (10)

24.	Explain the working principle of a magneto plasma dynamics (MPD) thruster with a neat sketch.

(OR)

25.	Explain in detail about electro static and electro dynamic rockets with neat sketch.