Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	PRINCIPLES OF CHEMICAL ENGINEERING		      	     Time : 3 hours

Subject Code:	09BT203								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	S.T.P means __________.

2. 	Write the S.I units of Viscosity and density.

3. 	Input=output is valid for __________.

4. 	A recycle ratio is defined as __________.

5. 	Fluids may be classified into __________, __________.

6.  	Friction factor in case of pipe flow depends on __________ and __________.

7. 	List out the fluid flow measurement devices.

8. 	Which is the most accurate liquid flow control device?

9. 	What do you meant by Cavitation?

10. 	What are the main parameters required for the selection of pump?

PART – B  (5 x 3 = 15 MARKS)

11. 	Define: Molarity, Molality, Normality.

12. 	Define: limiting reactant and conversion %.

13. 	What is the significance of Reynolds number?

14. 	What is the difference between an orifice meter and venturimeter?

15. 	Differentiate between the centrifugal pump and reciprocating pump.

PART – C  (5 x 15 = 75 MARKS)

16. 	a.	Air has 21% and 79% of oxygen and Nitrogen by volume respectively. What is the 	average molecular weight?                                                                              			    (5)

b.	Calculate the average molecular weight of a flue gas having the following composition by volume, CO2=13.1%, O2= 7.7%, and N2=79.2%. Calculate the i) Average  molecular weight of gas mixture ii) Density  of gas mixture at 303K and 101.32Kpa. 					  (10)

(OR)

17.  Cracked gas from a petroleum refinery contains CH4=45%, C2H6 =10%, C2H4=25%,       		C3H8=7%,C3H6=8%, and C4H10=5% by volume. Calculate

i) mole fraction, ii)The average molecular weight of  gas mixture, iii)the Composition by weight, iv)the specific gravity of the mixture taking average molecular weight of air as 28.84.

18. It is required to make 2000kg mixed acid containing 60% H2SO4 ,  32.% HNO3 and 8%         Water by blending (a) the spent acid containing 11.3%   HNO3, 44%  H2SO4 and 44.3 Water (b) aqueous 90% HNO3 and (c) aqueous 98% H2SO4. All percentages are by weight .Calculate the amount of a, b, and c are to be mixed.

(OR)

[P.T.O]

19.	a.	Calculate the heat that must be removed in cooling 32 kg of oxygen from 488 K to 313K 	Using Cp0 data. Cp = a+bT+CT2+dT3    ( KJ/ KmolK.)

| Gas | A | bx103 | cx106 | dx109 |
| --- | --- | --- | --- | --- |
| Oxygen | 26.025 | 11.75551 | -2.3426 | -0.5623 |

(8)

b.	Calculate the standard  heat of reaction, Δ H2O of the following reaction.

CaC2(solid) +2H2O (liquid) = Ca(OH)2(solid)+ C2H2 (g)

∆Hf   of  CaC2(solid) = -15000 cals, CO2 =  - 393. 51 KJ/Mol,  H2O (liq) = - 68,317 cals,

Ca(OH)2(solid) =  -235800 cals, and C2H2 (g)  = +54194 cals.                  			    (7)

20. 	a.	Describe the various types of fluids and its properties.                                   			    (8)

b.	Sulphuric acid of specific gravity1.3 is flowing through a pipe 6 cm inner diameter. An  orifice of 1 cm diameter  if fitted to pipe and the differential pressure  recorded by a mercury manometer is 10 cm. Assume that the leads to the manometer  are filled with acid. Calculate the flow rate of acid in kilogram per minutes. Use the value of Cd=0.61.	    (7)

(OR)

21.  Derive the continuity equation for the streamline flow with neat sketch.

22. 	a.	Explain the working principle of orifice meter.                                              			    (8)

b.	A sharp – edged orifice with an aperture of 20 mm diameter is inserted in a 50 mm i.d

pipe line carrying a raw material from the neighbouring factory. What Charges are to be

paid per day  to the factory for the chemical received, if the material costs Rs. 2 per kg?

The following data  are available.

Average viscosity of raw materials= 7 c.p, Average density =1.3gm/cc, Manometer

connected across the orifice reads 10 cm of Hg. Assume orifice coefficient is 0.61 .

(7)

(OR)

23.	Explain in details about flow control valves with neat sketch.

24. 	Describe the characteristics curves of centrifugal pump with diagram.

(OR)

25. 	Write short notes on: Fan, Blower and compressor with neat sketch.