Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	LINEAR INTEGRATED CIRCUITS AND APPLICATIONS													      	     								      Time : 3 hours

Subject Code:	09EC220								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	List the two commonly used methods for obtaining Integrated Circuit Capacitors.

2.	What is meant by an Integrated Circuits?

3.	If the gain of a closed-loop inverting amplifier is  -3.9, with an input resistor value of 1.6 kΩ, what value of feedback resistor is necessary?

4.	What is meant by slew rate in op-Amps?

5.	A zero crossing detector is a comparator with

a.	Vin =0		b.	Vcc= 0		c.	Vref=0

6.	State the Barkhausen criteria for oscillation.

7.	The value Q for a narrow band pass filter is ________ and for wide band pass filter is ________.

8.	The damping coefficient of Chebyshev filter is_________.

9.	Define capture range of a PLL.

10.	Mention the two widely used integrating type ADCs converters.

PART – B  (5 x 3 = 15 MARKS)

11.	List down the three different IC package Configuration.

12.	R1 = 10k, Rf = 100k, Vi = 1V. A load of 25 k is connected to the output terminal. Calculate I1, V0 and IL.

13.	Find the frequency of oscillation of triangular wave generator, if R1 = R2 = R3 = 1KHz, C=0.1µF.

14.	For a mono-stable multi-vibrator using 555 Timer, determine the value of C. Given R = 100 k and time delay is 100msec.

15.	Mention any three applications of PLL.

PART – C  (5 x 15 = 75 MARKS)

16.	Describe the basic planar processes involved in fabrication of Integrated circuits.

(OR)

17.	With Necessary diagrams, explain the four different methods available for fabricating Integrated Circuit Resistors.

[P.T.O]

18.	Explain the following electrical parameters related to ideal op-amp:

a. Input bias current	b. Input offset current	   c. Input offset voltage  d. Thermal drift

(OR)

19.	How can the multiplier IC be configured as a divider? Draw the diagram and derive the expression for the divider circuit.

20.	What is an astable multivibrator? Derive the frequency for an astable multivibrator.

(OR)

21.	Explain the working of an RC phase shift oscillator with a neat circuit. Derive the frequency of oscillation.

22.	a.	Design a wide band pass filter having fl = 400Hz, fh = 2 kHz and pass band gain of 4. Find 	the value of Q.																  (10)

b.	Why active filters are preferred? And classify the commonly used filters. 			    (5)

(OR)

23.	Sketch the functional diagram of a 555 timer and explain how it works as an Astable Multivibrator. Derive the expression for the Time period.

24.	With relevant block diagram, explain the functionality of a PLL.

(OR)

25.	Draw the circuit diagram of a 3 bit R-2R ladder Digital to analog converter and find the output voltage for the digital data 100.