Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	BASICS OF AEROSPACE ENGINEERING			      	      Time: 3 hours

Subject Code:	10AE202						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The first fixed-wing flying machine concept with separate system for lift and propulsion was set down by ________.

2.	Name any TWO public players in aviation industry.

3.	A wing with _______ is good for transport aircraft to reduce Dinduced .

4.	Define Avionics.

5.	Write down any one advantage of biplane.

6.	Show the degrees of freedom of an aircraft.

7.	Define false spars.

8.	What are the materials used in aircraft wings?

9.	What is meant by hypergolic propellants?

10.	Write down the fundamental thrust equation for jet propulsion.

PART – B  (5 x 3 = 15 MARKS)

11.	Differentiate hot air balloon from gas balloon.

12.	What are the types of flight controls?

13.	Write down six factors that affect lift produced by an airfoil.

14.	List out the advantages of semimonocoque fuselage.

15.	Define TSFC.

PART – C  (5 x 15 = 75 MARKS)

16.	Write Short notes on

a.	Ornithopter		b.	Glider 		c.	Rotary wing aircraft

(OR)

17.	Explain in detail about the developments in Aerodynamics over a year.

18.	Explain the major aircraft components and its functions with neat sketch.

(OR)

19.	Discuss the various types of flight control system.

20.	Classify the atmosphere according to air temperature and derive the relationship between pressure and temperature.

(OR)

21.	Discuss the airfoil terminology with neat sketch.

22.	Discuss the types of fuselage construction used in aircraft with neat sketches.

(OR)

23.	a.	Discuss the procedures for the selection of aircraft materials.						    (5)

b.	Describe the application of titanium alloys for the aerospace industry.				  (10)

[P.T.O]

24.	Explain the working principle of turboprop engine with a neat sketch.

(OR)

25.	A two-stage rocket has the following masses:1st-stage propellant mass 120,000kg, 1st-stage dry mass 9,000 kg, 2nd-stage propellant mass 30,000 kg, 2nd-stage dry mass 3,000 kg, and payload mass 3,000 kg. The specific impulses of the 1st and 2nd stages are 260 s and 320 s respectively. Calculate the rocket's total V.