Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	ELECTRIC CIRCUIT ANALYSIS					      	     Time : 3 hours

Subject Code:	10EC202						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Calculate the total voltage across the terminal A-B.

2.	The current flowing through each resistance is different in ___________circuit.

3.	A ______is a point in a network in which two or more elements have a common connection.

4.	What happens to the circuit?

5.	Determine the value of load resistance RL when the load resistance draws more power.

6.	________ is defined as a loop which does not contain any other loops within it.

7.	Which theorem states that any linear bilateral network can be replaced by a single Current source, IN, in parallel with single impedance, Zth?

8.	Which theorem states, that the response of a linear circuit with multiple sources is given by algebraic sum of responses due to individual sources acting alone?

9.	Write down the current transient equation for DC response of an R-L circuit.

10.	___________& ____________ are the complete response of the circuit.

PART – B  (5 x 3 = 15 MARKS)

11.	Find the equivalent resistance across the terminal ab.

[P.T.O.]

12.	Determine the total impedance, current I for the series circuit consisting of R=10Ω, L=0.1H, C=10µF and Vs=50V, 50Hz.

13.	The number of turns in an ideal transformer are N1=10 & N2=100. Determine the primary impedance if the secondary load is 1000Ω.

14.	State Millman’s theorem.

15.	Write a short note on forced response, transient response of a circuit.

PART – C  (5 x 15 = 75 MARKS)

16.	Find the total current flow in the circuit and give their equivalent resistance for the circuit given below. (All values are in ohms)

(OR)

17.	Obtain the value of resistance R2 when the power absorbed by 10Ω resistance is 160W.

18.	Determine the total impedance, current I, phase angle θ, and the voltage across each elements.

(OR)

19.	Draw the circuit of 3-phase balanced star connected load with wattmeter for power measurement and also prove that two wattmeter are sufficient to measure 3-phase power.

20.	Find the node voltages for the given circuit; also find the voltage drop across 3 ohm resistor. (All resistance are in ohms)

(OR)

21.	Apply mesh analysis to find current flowing through 2 Ω resistor.

22.	Verify the reciprocity theorem across the terminals for the network shown. VS=20V , R1=2Ω ,	R2=2Ω , R3=3Ω , R4=2Ω & R5=2 Ω.

(OR)

23.	a.	Find the current through 24Ω resistor using Thevenin’s theorem.					  (10)

b.	State Maximum power transfer theorem and derive the expression how RS=RL.		    (5)

24.	A series RL circuit with R=30Ω and L=15H has a constant voltage V=60V applied at t=0. Determine the current I, the voltage across resistor and the voltage across the inductor.

(OR)

25.	A series RC circuit consists of 10Ω and capacitor of 0.1F. A constant voltage of 20V is applied to the circuit at time t=0. Obtain the current equation also determine the voltage across the resistor and capacitor.