Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	DIGITAL SIGNAL PROCESSING					      	     Time : 3 hours

Subject Code:	10EE202 / 12EE225 / EE260		 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	If x(n) = x(-n) then the signal is called __________.

2.	What is a random signal?

3.	Define impulse response.

4.	State the condition for performing Circular convolution.

5.	In Butterworth filter, the order, N is given by __________.

6.	Why FIR filter is stable always?

7.	Define Input Quantization error.

8.	List the advantages of floating point arithmetic.

9.	What do you mean by Pipelining in DSP processor?

10.	Expand MIPS.

PART – B  (5 x 3 = 15 MARKS)

11.	Convolve the two sequences x(n) = {1,2,3} and h(n) = {5,4,6,2} and fill up the blank

y(n) = {__, 14 ,__ ,__ ,__ , 6}

12.	Draw and explain the basic butterfly diagram or flow graph of DIF radix-2 FFT.

13.	What is warping effect? How can it be avoided?

14.	Compare floating point arithmetic and fixed point arithmetic.

15.	Draw the configuration of Multiplier Accumulator Unit and explain briefly.

PART – C  (5 x 15 = 75 MARKS)

16.	Classify the types of signals.  Sketch and represent mathematically any five signals.

(OR)

17.	a.	With a neat block diagram, explain the typical DSP system.						    (9)

b.	Let x(n) ={1,3,0,2,1} and h(n) ={ 2,3}											    (6)

Perform linear convolution and Circular convolution of the given two sequences.

18.	Find the DFT of  sequence x(n) = {1,2,3,4,4,3,2,1} using DIF algorithm.

(OR)

19.	Compute IDFT of the sequence X(k) = {7, – 0.707 – j0.707,  – j, 0.707 – j0.707, 1,

0.707 + j0.707,  j,  – 0.707 + j0.707} using DIT algorithm.

20.	Describe the method of design of IIR filters using bilinear transformation.

(OR)

21.	Describe the method design of FIR filter using Hamming window with N=2.

22.	What is Quantization? Explain the different ways of performing quantization and the errors associated with them.

[P.T.O]

(OR)

23.	With illustrations, explain the different ways of representing numbers in digital hardware.

24.	Explain how extended parallelism can be achieved using SIMD, VLIW and super scalar processing.

(OR)

25.	a.	Compare Van-Neumann and Harvard architectures with necessary diagrams.   		    (8)

b.	What are the various factors that should be considered while selecting a DSP processor? 	Explain.							           										    (7)