Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	PROCESS DYNAMICS AND CONTROL			      	     Time : 3 hours

Subject Code:	10EI208/12EI219								       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.   Draw the Piping & Instrumentation diagram of Pneumatic signal and electrical signal.

2.   Write the formula for degrees of freedom.

3.   __________ is the other name of Proportional-Integral Controller.

4.   What is Offset?

5.   __________ is the best suitable controller for a slow process.

6.   What are the parameters required to design a best controller?

7.   What is a final control element?

8.   List the different types of Actuators.

9.   Expand CSTR.

10. ________ is the controller which has two measurements and one manipulated variable.

PART – B  (5 x 3 = 15 MARKS)

11. 	Draw the P&ID diagram of Hydraulic Signal, Butterfly Valve and Temperature recorder controller mounted in the control room.

12. 	Write short notes on ON-OFF Controller.

13. 	Write short notes on ISE, IAE and ITAE.

14. 	With a neat sketch, explain the construction of Control Valve.

15. 	Distinguish between feedback and feed forward control.

PART – C  (5 x 15 = 75 MARKS)

16.  	Derive the mathematical model for the following first order systems:

a. 	Mixing Process. 																    (7)

b. 	Mercury Thermometer.														    (8)

(OR)

17. 	Analyze the two tank interacting liquid level system with its mathematical model.

18. 	Explain P, PI, PID control modes in detail with suitable response graphs.

(OR)

19.  	Explain the general procedure for selecting the type of Feedback Controller. Also elaborate the selection of type of controller for various processes like liquid level, flow, Pressure and temperature processes.

20. 	Elaborate the controller tuning procedure and tuning rules of Cohen-Coon method.

(OR)

21. 	Explain in detail about the procedure and tuning rules of Ziegler Nichols tuning method.

22.  a.	Explain the need for valve positioner with suitable example.             				    (4)

b.  	With a neat sketch, explain how cavitation is formed in control valves.				    (5)

c.  	Describe about the working of any 2 types of Control valves.						    (6)

[P.T.O]

(OR)

23.  	Elaborate the working of various types of Pneumatic Actuators with necessary diagrams.

24.  	Explain the following in detail.

a.  	Cascade Control.																(5)

b.  	Ratio Control.																(5)

c.  	Split Range Control.															(5)

(OR)

25.  With necessary diagrams, Explain the various Control configurations of Top product and

Bottom Product composition in Distillation Column.