Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	MODERN CONTROL SYSTEMS					      	     Time : 3 hours

Subject Code:	10EI218											       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	In an open loop system the control action is dependent on output. (TRUE/ FALSE)

2.	What is source node in SFG?

3.	What is impulse input?

4.	For a type 0 system, __________ error coefficient is constant.

a.	Position        b. 	Velocity            c. 	Acceleration

5.	Define Resonant frequency.

6.	State Nyquist stability criterion.

7.	In rootlocus method ________ is assumed to be the parameter which is varied from 0 to ∞.

8.	Define centroid.

9.	Define state of a system.

10.	A MIMO system can be analysed using state variable representation. (TRUE/FALSE)

PART – B  (5 x 3 = 15 MARKS)

11.	Write short description on potentiometers.

12.	A unity feedback system with open-loop transfer function G(s) = 4/ [s(s + p)] is critically damped. Find the value of the parameter p.

13.	Draw the magnitude and phase plot for an integral factor which has a pole at origin.

14.	State the necessary and sufficient condition for stability.

15.	Define controllability and observability.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Find the transfer function of given block diagram, using block reduction techniques.	    (7)

[P.T.O]

b.	Obtain the transfer function of the Signal Flow Graph given.						    (8)

(OR)

17.	a. 	Find the transfer function of the given mechanical rotational system.	   			    (8)

b. 	With a neat diagram explain the working of two phase ac servomotor. 				    (7)

18. 	a. 	Derive the response of undamped second order system for unit step input.  			  (10)

b. 	Classify the second order system in terms of value damping ratio and the roots of characteristic equation.														    (5)

(OR)

19.	The open loop transfer function of a system with unity feedback gain is given as Gs=25s2+6s+25. Determine the damping ratio, maximum overshoot, rise time and peak time, settling time for 2% tolerance.

20. 	A unity feedback control system has GS=10ss+10.02s+1. Draw the bode plot. Determine gain margin and phase margin.

(OR)

21. The open loop transfer function of a unity feedback system is given by

GS=1ss+12s+1.

Sketch the polar plot and determine the gain margin and phase margin.

22. 	For a unity feed back system,

GS=KS+2S+4S2+6S+25

Apply Routh's criterion and discuss the stability of the closed loop system as a function of K. Determine the value of K which will cause sustained oscillations in the closed loop system. What are the corresponding frequencies of oscillations?

(OR)

23. 	Sketch the complete root locus of system having the open loop transfer function

GS=KS+9SS2+4S+11

24. 	A control system is described by the differential equation

d3y(t)dt3+6d2y(t)dt2+11dytdt+10y=8ut

Where y(t) is the observed output and u(t) is the input. Express the system in the state variable form

x=Ax+Bu

y=Cx+Du

(OR)

25. 	A system is represented by the state and output equations given below. Find the poles of the system.

X'=−3−2−1−2X+11u(t)

Y=12X