Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May - 2016

Subject Title :	Mechanics of Machines - II							      	     Time : 3 hours

Subject Code:	10ME201 /12ME223 /ME208 /ME255 			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What are the conditions for static equilibrium of a body?

2.	State D’Alembert’s principle.

3.	What is the function of a flywheel?

4.	Define coefficient of fluctuation of speed in a flywheel.

5.	Crankshaft is an example of  balancing in -----------planes.

6.	What is hammer blow?

7.	What is the equivalent stiffness of two springs of stiffness k1 and k2  connected in series?

8.	What is the condition for whirling speed of a shaft?

9.     Define torsional stiffness.

10.	What is the value of amplitude at a node in a torsional vibration system?

PART – B  (5 x 3 = 15 MARKS)

11.	Draw a slider crank mechanism and indicate various forces acting on the links and joints.

12.	Draw the turning moment diagram for a four stroke IC engine.

13.	Briefly explain primary unbalanced force and secondary unbalanced force.

14.	Obtain the natural frequency of a spring mass system with spring  stiffness 10000 N/m and mass 10 kg.

15.	Derive the relation between lengths and diameters of a stepped shaft and an equivalent shaft.

PART – C  (5 x 15 = 75 MARKS)

16.	The four-bar linkage has crank 2 driven by torque T;  an external load F=66 N at an angle of

42º on link 4. For the particular position of the linkage shown find all the constraint forces and

their reactions necessary for this link to be a position of equilibrium.

T

B

A

2

60º

4

100º

10º

Q

F

3

O4

O2

O4O2=110cm

AO2=55cm

BO4=61.6cm

AB=72.6cm

QO4=41cm

42º

(OR)

17.	The stroke of a steam engine is 600mm and the length of the connecting rod is 1.5m. The crank

rotates at 180r.p.m. Determine:  (i) velocity and acceleration of the piston, when the crank has

travelled through an angle of 40⁰ from inner dead centre, and (ii) the position of the crank for

zero acceleration of the piston.

[P.T.O]

18.	The turning moment diagram for a multi-cylinder engine has been drawn to a scale  	1mm=4500N-m vertically and 1mm = 2.4º horizontally. The intercepted areas between the 	output torque curve and the mean resistance line, taken in order from one end, are as follows: 	+342, -23, +245, -303, +115, -232,+227 and -371 mm2 , when the engine is running at a speed 	of 150r.p.m. If the mass of the flywheel is 1000kg and the total fluctuation of speed is not to 	exceed ± 3% of the mean speed, find the minimum value of the radius of gyration.

(OR)

19.	The turning moment diagram for a multi-cylinder engine has been drawn to a scale of 1 mm to

500 N-m torque and 1 mm to 6° of crank displacement. The intercepted areas between output

torque curve and mean resistance line taken in order from one end, in sq. mm are – 30, + 410,

– 280, + 320, – 330, + 250, – 360, + 280, – 260 sq. mm, when the engine is running at

800 r.p.m. The engine has a stroke of 300 mm and the fluctuation of speed is not to exceed ±

2% of the mean speed. Determine a suitable diameter and cross-section of the flywheel rim for

a limiting value of the safe centrifugal stress of 7 MPa. The material density may be assumed 	  as 7200 kg/m3. The width of the rim is to be 5 times the thickness.

20. 	Four masses m1, m2, m3 and m4 are 200 kg, 300 kg, 240 kg and 260 kg respectively. The corresponding radii of rotation are 0.2 m, 0.15 m, 0.25 m and 0.3 m respectively and the angles

between successive masses are 45°, 75° and 135°. Find the position and magnitude of the balance mass required, if its radius of rotation is 0.2 m.

(OR)

21. 	A shaft carries four masses A, B, C and D of magnitude 200 kg, 300 kg, 400 kg and 200 kg

respectively and revolving at radii 80 mm, 70 mm, 60 mm and 80 mm in planes measured from

A at 300 mm, 400 mm and 700 mm. The angles between the cranks measured anticlockwise are

A to B 45°, B to C 70° and C to D 120°. The balancing masses are to be placed in planes X and

Y. The distance between the planes A and X is 100 mm, between X and Y is 400 mm and        between Y and D is 200 mm. If the balancing masses revolve at a radius of 100 mm, find their

magnitudes and angular positions.

22.  Mass of a single degree damped vibration system measures 6 kg and makes 25 free oscillations

in 11 seconds. The amplitude of vibration reduces by 30%  of its initial value after 5 oscillation.

Determine (i) Stiffness of spring  (ii) Logarithmic decrement  (iii) Damping factor  (iv) Critical

damping  coefficient  and  (v) actual damping coefficient.

(OR)

23.	A shaft 50 mm diameter and 3 metres long is simply supported at the ends and carries three 	loads of 1000 N, 1500 N and 750 N at 1 m, 2 m and 2.5 m from the left support. The Young's 	modulus for shaft material is 200 GN/m2. Find the frequency of transverse vibration using  	Dunkerly’s method.

24.	A steel shaft 1.6m long is 95mm in diameter for the first 0.6m of its length, 60mm in diameter for the next 0.5 m of the length and 50mm in diameter for the remaining length. The shaft carries two rotors at two ends, the first having a mass of 800kg and 0.85m radius of gyration located at the 95mm diameter end and the second having a  mass of  600kg and 0.55m radius of gyration located at the other end. Determine the  location of the node and the natural frequency of free torsional vibration of the system. The modulus of rigidity of shaft material is 80GN/m2 .

(OR)

25.	A centrifugal pump rotating at 400 r.p.m is driven by an electric motor at 1200 r.p.m through a single stage reduction gearing. The moments of inertia of the pump impeller and the motor are 1500 kg. m2 and 450 kg. m2 respectively. The lengths of the pump shaft and the motor shaft are 500 and 200 mm, and their diameters are 100 mm and 50 mm respectively. Neglecting the inertia of the gears, find the frequency of torsional oscillations of the system. Take modulus of rigidity C = 85 GN/m2.