Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	COMPUTATIONAL FLUID DYNAMICS			      	     Time : 3 hours

Subject Code:	10ME203								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is the philosophy followed by the basic equations of fluid motion?

2.	What is the continuity equation for viscous flow?

3.	What is the main advantage of FVM over FDM?

4.	Define control surface.

5.	What is hybrid scheme?

6.	Write the condition for using power-law scheme.

7.	SIMPLE algorithm gives the method of calculating _________and __________.

8.	Write the advantage of explicit method.

9.	Write the advantages of mixing length model.

10.	Write the general transport equation for combustion modelling.

PART – B  (5 x 3 = 15 MARKS)

11.	What is time average scalar transport equation?

12.	Write the example for source term linearization.

13.	What is the accuracy of central differencing scheme?

14.	Write the velocity correction equation of SIMPLEC.

15.	List out the most common turbulence models.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the effect of turbulence on time averaged Navier-stokes equations.

(OR)

17.	Classify the partial differential equations (PDEs) and explain the role of characteristics in hyperbolic equations.

18.	Consider the source-free heat conduction in an insulated rod whose ends are maintained at the constant temperatures of 100oC and 500oC. Calculate the steady state temperature distribution in the rod. Thermal conductivity k equals 1000 W/mK, cross-sectional area A is 10 x 10-3 m2.

(OR)

19.	Discuss the formulation of finite volume method for three dimensional diffusion problems.

20.	Explain the formulation of finite volume method for steady one dimensional convection and diffusion.

(OR)

21.	Discuss about upwind differencing scheme and write its accuracy.

[P.T.O]

22.	Explain in detail about Alternating Direction Implicit (ADI) method.

(OR)

23.	Describe the SIMPLER algorithm with a flow chart.

24.	Explain k-ε Model in detail and also write its advantages and disadvantages.

(OR)

25.	Explain in detail about simple chemical reacting system.