Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	FINITE ELEMENT ANALYSIS						      	     Time : 3 hours

Subject Code:	10ME205						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	List the application of finite element in Mechanical engineering.

2.	Define weighted residual method.

3.	Write the purpose of Pascal’s triangle.

4.	Define the term ‘Finite element’.

5.	Write the interpolation polynomial equation for 2D bilinear rectangular element.

6.	Define local co-ordinate system.

7.	Sum of all shape function is ___________.

8.	Write the advantages of using higher order elements.

9.	Define Incompressible flow.

10.	Write the stiffness matrix for 1D composite wall subjected to convective heat loss at left side.

PART – B  (5 x 3 = 15 MARKS)

11.	Define Boundary value problem.

12.	Write the properties of stiffness matrix.

13.	Define inter-element compatibility.

14.	Define r, p and h methods in Finite element.

15.	Write the general stiffness matrix and force vector for 1-D heat transfer in a fin problem.

PART – C  (5 x 15 = 75 MARKS)

16.	Solve the differential equation for a physical problem expressed as

d2ydx2+100=0; 0 £x £ 10

with the boundary conditions  y (0) = 0 and y(10) = 0 using point collocation method, Galerkin’s method, least square method and sub domain method. Assume suitable trial function.

(OR)

17.	Consider the differential equation for the problem as, d2ydx2+400x2=0; 0 £	x £ 1 with the boundary conditions  y (0) = 0 and y(1) = 0, find the solution of the problem using two term trial function and using point collocation and sub domain methods. Assume suitable trial function.

18.	a.	Derive the shape function for 1D linear element.									(10)

b.	List the properties of shape function.											  (5)

(OR)

[P.T.O.]

19.	Calculate the deflection and slope for the beam fixed at one end and supported by a roller at other end (shown in Figure). Assume E = 2x107 N/cm2, I = 2500cm4, L=1000cm and P = 20 kN

20.	Determine three points on the 500C contour line for the rectangular element shown in Figure. The nodal values are T1=420C, T2= 540C, T3= 560C and T4= 460C. Nodal coordinates are node1 (5,3), node2 (8,3), node3 (8,5) and node4 ( 5,5).

S

x

Y

2(8,3)

1(5,3)

3 (8,5)

4(5,5)

t

(OR)

21.	For the triangular element shown in figure, the nodal values of displacement are

u1 = 0.001; 	 u2 = 0.003; 	u3 = –0.002

v1 = –0.004;	 v2 = 0.002;	v3 = 0.005

Find the displacement u, v at point P (2, 5) within the element. (All Dimensions are in mm)

u



(1,1) 1

2 (3,4)

3 (2, 7)

v

pu

22.	Derive the shape functions for 1D cubic element.

(OR)

23.	For the plane stress element shown in figure, the nodal displacements are u1 = 2 mm; v1 =1mm, u2 = 1 mm ; v2 = 1.5 mm, u3 = 2.5mm ; v3 =0.5 mm.

Determine the element stresses. Assume plate thickness = 10 mm, Young’s modulus = 200GPa and Poisson’s ratio = 0.3. The co-ordinates are given in units of millimeters.

1 (100, 100)

2 (400, 100)

3 (200,400)

x

y

24.	A wall of an industrial oven consists of three different materials. The first layer is composed of 5 cm of insulating cement with a clay binder that has a thermal conductivity of 0.08 W/m K. The second layer is made from 15 cm of asbestos board with a thermal 	conductivity of 0.074 W/m K. The exterior consists of 10 cm common brick with a thermal 	conductivity of 0.72 W/m K. The inside (inlet) wall temperature of the oven is 200°C, and 	the outside air is 30°C with a convection coefficient of 40W/m2K (Convective heat loss 	occurs at right side). Determine the temperature distribution along the composite wall. 	Consider area of the element as 1m2.

(OR)

25.	Derive the stiffness matrix for one dimensional fluid flow through a porous medium.