Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	DIGITAL SIGNAL PROCESSING					      	     Time : 3 hours

Subject Code:	10MT203										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Draw the unit step signal.

2.	Name any two properties of DTLTI system.

3.	List the advantages and applications of FFT.

4.	Mention the number of complex multiplications required for calculating the DIT FFT.

5.	Draw the magnitude response of a type II Chebychev filter.

6.	What are the four realization techniques?

7.	Mention any two characteristics of FIR filter.

8.	What are the desirable characteristics of windows?

9.	Draw the quantisation process model.

10.	What are the two types of quantisation?

PART – B  (5 x 3 = 15 MARKS)

11.	Sketch y(n) for –3 < n < 4, if y(n) = 2 δ(n + 6) + δ(n) + u (n –1).

12.	What are the computational savings in evaluation of DFT using radix-2 FFT algorithm.

13.	What is "Many to One Mapping”?

14.	Define phase delay and group delay.

15.	Explain quantisation of floating point numbers.

PART – C  (5 x 15 = 75 MARKS)

16.	State sampling theorem. Explain the sampling process and the effects of undersampling with a neat diagram.

(OR)

17.	Perform linear convolution of x(n) = { 1, 2, 3, -1, -2, -3, 4, 5, 6} and h(n) = {2, 1, -1} using overlap save method.

18.	a.	Compute DTFT OF x(n) =  a  ׀n ׀ , ׀a ׀< 1 										(10)

b.	Explain the procedure for computing the IDFT using FFT algorithm.				(5)

(OR)

19.	Find the DFT of x(n) = 2sin(nπ/4) using DIT -  FFT algorrithm.

20.	Design a digital LPF using the Bilinear transformation method to satisfy the following  characteristics

a.	monotonic stop band and pass band

b.	-3.01 dB cut-off frequency of 0.5 π rad c) magnitude down atleast 15 dB at 0.75 π

(OR)

21.	Realize the transfer function using Direct Form I and II realization

H(z)=1+12z−1(1−z−1+14z−2)(1−z−1+12z−2)

[P.T.O]

22.	Explain magnitude and phase response of FIR filters.

(OR)

23.	Design a FIR digital filter to approximate an ideal LPF with pass band gain of unity, cut-off frequency of 850 Hz and working at a sampling frequency of 5000 Hz.  The length of the impulse response should be 5. Use Hamming window.  Realize the same using minimum number of multipliers.

24.	Discuss in detail about the representation of numbers in registers.

(OR)

25.	Explain the following

a.	Truncation error.																    (5)

b.	Co-efficient quantization error. 												    (5)

c.	Limit cycle oscillation. 														    (5)