Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	AERODYNAMICS									      	     Time : 3 hours

Subject Code:	11AE205/12AE204				 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is unit of density?

a.	kg/m3        	b.	N            	c.	Nm            		d.	 kg m/s

2.	What is unit of momentum?

a.	kg/m3        	b.	N            	c.	Nm            		d.	kg m/s

3.	Equi-potential lines and streamlines are

a.	parallel      	b.	orthogonal   	c.	at acute angle    	d.	at obtuse angle

4.	Bernoulli equation relates

a.	velocity and pressure    		b.	velocity and temperature

c.	pressure and density  			d.	velocity and density

5.	Define pressure coefficient.

6.	What is uniform flow?

7.	What is source flow?

8.	Biot-Savart law relates vortex strength and position vector to

a.	induced velocity    			b.	density

c.	pressure  					d.	temperature

9.	Kutta condition specifies velocity for airfoil at the

a.	leading edge   b.	trailing edge  c.	quarter chord point  	d.	at mid-point of chord

10.	According to thin airfoil theory, the slope of lift coefficient curve is

a.	π        		b.	 2π        		c.	 π/2         			d.	2/π

PART – B  (5 x 3 = 15 MARKS)

11.	Write continuity equation for incompressible 3D flow in Cartesian coordinate system.

12.	Define Streak line.

13.	Define velocity potential.

14.	State Kutta-Joukowskii theorem.

15.	Differentiate between symmetric and cambered airfoil.

PART – C  (5 x 15 = 75 MARKS)

16.	The velocity potential for a flow condition is given by

Φ=Ax2−y2

- Find the velocity components for this flow.										    (6)

- Find the corresponding stream function Ψ.										    (9)

(OR)

17.	The stream function Ψ for a flow condition is given by

Ψ=2Axy

a.	Find the velocity components for this flow.										    (6)

b.	Find the corresponding velocity potential Ф.										    (9)

[P.T.O]

18.	For a velocity field, in cylindrical coordinate system, given by

ur,θ=C1−1r2cosθ

vr,θ=−C1+1r2sinθ

where C is an arbitrary non-zero constant. Does this satisfy continuity equation for incompressible flow?  If yes, find the pressure distribution for satisfying the equation of motion.

(OR)

19.	For a velocity field given by

ux,y=Bx2−y2x2+y22

vx,y=2Bxyx2+y22

a.	Find out whether continuity equation is satisfied for incompressible flow or not.            (9)                                               	b.	If continuity equation is satisfied, find the pressure distribution so as to satisfy the 				momentum equation.          													    (6)

20.	For a combination of source and sink of the same strength Q at (–c,0) and (c,0) respectively, derive the stream function Ψ.

(OR)

21.	For a combination of source and sink same strength Q at (–c,0) and (c,0) respectively in a uniform stream with velocity U along x axis, find the stagnation points. Assume that stagnation points will be on the x-axis.

22.	For a thin aerofoil of chord length 2l in a uniform flow field with velocity U and density ρ, the velocity on the upper and lower surface is given by  uu=U11+xl2and  ul=U11−xl2  with origin being taken at the mid point of chord. Using the Kutta Joukowskii theorem, estimate the lift of the aerofoil. Neglect the contribution of velocity component along y.

(OR)

23.	The Camber line of a thin airfoil is a circular arc given by

yc=4hxc1−xc. Find out the lift coefficient, zero lift angle and the vorticity distribution using thin airfoil theory.

24.	Using lifting line theory show that for wing with elliptical plan form without twist, circulation distribution along the span is also elliptic.

(OR)

25.	Show that for elliptic distribution of circulation, the induced downwash velocity due to trailing vortex is constant along the span of the wing.