Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	NETWORK ANALYSIS AND SYNTHESIS			      	     Time : 3 hours

Subject Code:	11EC202								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	In a transfer function the complex or imaginary poles and zeros must be_________.

2.	The driving point impedance is defined as the ratio of _________.

3.	If G(jω) =R(ω)+ j X(ω), express |G(jω)| and    G(jω) in terms of R(ω) and X(ω).

4.	What is complex locus?

5.	Draw the dual of the following circuit.

R

+

V

i

L

-

6.	Tree contains ________branches, if n is the number of nodes.

7.	Write down the condition for symmetry for Z-parameter of a network.

8.	Give the expression for the cut off frequency of constant K high pass filter.

9.	In a Hurwitz polynomial, all coefficients are__________.

10.	In RL driving point function, there is no ______as origin.

PART – B  (5 x 3 = 15 MARKS)

11.	Verify whether the following expression for driving point impedance is suitable for 	representing a passive one port network. Z(s)= s2+s+2s4+5s3+6s2 .

12.	For the given denominator polynomial of a network function, verify the stability of the network 	using Routh criterion. Q(S)= s3+s2+3s+8

13.	For the graph shown in figure, obtain the incidence matrix.

14.	Write the standard h-parameter equations.

15.	State the necessary and sufficient conditions for positive real function.

PART – C  (5 x 15 = 75 MARKS)

16.	Draw the pole zero diagram for the given network function I(s) and hence obtain  i(t).

I(s)=20s(s+5)(s+2)

[P.T.O]

(OR)

17.	For the network shown in figure, determine Z11(S), Z21(S) and G21(S).

18.	Obtain the complex locus plot for G21(jω), Z11(jω) and Z21(jω) for the given network.

(OR)

19.	Obtain the magnitude and phase plot for Z11(jω) and Z21(jω) for the given network.

20.	Draw the graph of the network shown in figure. Solve the network using tieset schedule method

12 V

2

6

6

4Ω

C

2

4Ω

B

D

A

8 V

6 V

(OR)

21.	Solve the network using cutset schedule.

5 V

+

-

2

1

1

1

2

22.	Find the Z parameters for the circuit shown in Figure.

(OR)

23.	Design a constant k high pass filter having a cut off frequency of 5000 Hz and a design impedance of 600 ohm. Draw (a) T section filter and (b)   section filter.

24.	Find the two Cauer realization of the given impedance function. Z(s)=4(s2+1)(s2+9)s(s2+4).

(OR)

25.	Check whether the given polynomial P(s) is Hurwitz or not.

P(s) = s5 + 3s3 + 2s