Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	SIGNALS AND SYSTEMS							      	     Time : 3 hours

Subject Code:	11EC203						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	A signal x(t) is called an energy signal if its energy is ________ and its power is __________.

2.	Is the system y(n) = n x(n), static or dynamic?

3.	The region of convergence of the Z-transform of a unit step function is __________.

4.	What is the condition for an LTI system having impulse response h(n) to be stable?

5.	Using what the periodic signals are represented, Fourier Series or Fourier Transform?

6.	Find the Fourier transform of the function x(t) = δ(t).

7.	If x(n) is of length N1 and h(n) is of length N2, what would be length of linear convolution sequence?

8.	The process of converting continuous time signals into discrete time signal is called _________.

9.	_________ of the system is the ratio of output of the system to the input signal.

10.	In DTFT, frequency domain is ___________ and periodic while time domain is discrete.

PART – B  (5 x 3 = 15 MARKS)

11.	Define unit impulse signal and unit step signal.

12.	Write any two properties of ROC of Z-transform.

13.	Write short notes on Dirichlet’s conditions for fourier transform.

14.	State sampling theorem.

15.	Find the laplace transform of  e−atu(t).

PART – C  (5 x 15 = 75 MARKS)

16.	For the signal x(t) shown, find the signals

a.	x(t - 2)																	    (3)

b.	x(2t + 3) 																    (4)

c.	x(3t/2) 																    	    (4)

d.	x(-t + 1) 																    	    (4)

(OR)

17.	a.	Check whether the system y(n) = x(n2) is causal or not. 							    (3)

b.	Check whether the system 	dydt+3ty(t)=t2x(t)is linear or not.						    (6)

c.	Determine whether the system y(t) = t x(t) is time invariant or not.					    (6)

[P.T.O]

18.	Find the convolution of x(t) and h(t) given as

x(t) = 1		0 ≤ t ≤ 2

= 0		otherwise

h(t) = 1		0 ≤ t ≤ 3

= 0 	       otherwise

(OR)

19.	Find the Z-Transform and ROC for the following signals

a.	 x(n) = u(n)		     				b.	x(n) = δ(n)

c.	xn=12nu(−n) 				d.	xn=122u(n)+13nu(n)

20.	Find Fourier series coefficients of half wave rectified sine function

(OR)

21.	State and prove any five properties of CTFT.

22.	Write in detail on impulse sampling and its implication on frequency spectrum.

(OR)

23.	a.	Give a detailed description on reconstruction of CT signal from its samples.			  (11)

b.	Determine the sampling frequency, fs needed for the signal xt=cosπ t+3sin2π t+sin4π t. 											    (4)

24.	Find the frequency response of the causal systems

a.	yn=12xn+xn−1+12xn−2												    (7)

b.	yn−14yn−1−38y(n−2)=x(n)+x(n−1)									    (8)

(OR)

25.	a.	Find the step response of the system 											    (7)

y(n)−34y(n−1)+18yn−2=x(n)

b.	Consider a discrete time LTI system with impulse response h(n)=12nun. Use Fourier Transform to determine the response of the system to the input xn−34nu(n).		    (8)