Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	DIGITAL ELECTRONICS							      	     Time : 3 hours

Subject Code:	11EC204 / EC284				 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	(64)10 = (          )2.

2.	Find the 1’s complement of 1011000.

3.	Draw the truth table of NAND gate.

4.	Write the expression for SUM for a Full adder.

5.	What are types of sequential circuits?

6.	Define latch.

7.	Draw the state diagram of a 3 bit binary up counter.

8.	Give the excitation table for D flip flop.

9.	PAL is a logic device with _________, _________ gates.

10.	Which is the fastest logic family?

PART – B  (5 x 3 = 15 MARKS)

11.	Reduce the Boolean expression x+x’y.

12.	Construct a half adder circuit.

13.	Draw a Mod -4 asynchronous counter.

14.	Differentiate state table and excitation table.

15.	Discuss the properties of TTL families.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Reduce the expression (A+B) (A’+C) (B+C). 									    (8)

b.	(875.12)10 = (  )8.   															      (2)

c.	(1110101)2  =  (  )10.  															      (3)

d.	Find the 2’s complement of 100101.											    (2)

(OR)

17.	a.	Convert binary 1110010 to hexadecimal and gray code.							    (5)

b.	Using K-map simplify the following Boolean function and obtain the minimal SOP.

Y= ∑m (0, 2, 3, 6, 7) + ∑d(8, 10, 11, 15).										  (10)

18.	a.	Implement the following Boolean function using NAND gates

F (x,y,z) = ∑m (1,2,3,4,5,7)													    (7)

b.	Design a 1 bit Full adder.														    (8)

(OR)

19.	a.	Implement the following functions using De mux

F1 (A,B,C) = ∑m (0,3,7)

F2 (A,B,C) = ∑m (1,2,5)  														    (8)

b.	Design a 2 x4 Decoder.														    (7)

[P.T.O]

20.	Draw the state diagram of the state table given below. Reduce the number of states in the state table, tabulate the reduced state table and draw its corresponding state diagram.

| Present State | Next State |  | Output |  |
| --- | --- | --- | --- | --- |
|  | x=0 | x=1 | x=0 | x=1 |
| a | f | b | 0 | 0 |
| b | d | c | 0 | 0 |
| c | f | e | 0 | 0 |
| d | g | a | 1 | 0 |
| e | d | c | 0 | 0 |
| f | f | b | 1 | 1 |
| g | g | h | 0 | 1 |
| h | g | a | 1 | 0 |

(OR)

21.	Draw the circuit of D Flip Flop. Write its Characteristic table, Characteristic equation, truth table and excitation table.

22.	Design a Mod 10 Ripple Counter

(OR)

23.	Design a 3 bit up/down counter using D flip flop.

24.	Y0 = AB+A'CB'

Y1 = AB'C+AB+AC'

Y2 = AB+BC+CA

Implement the above functions on

a.	PLA architecture.  								    							    (8)

b.	PROM architecture.															    (7)

(OR)

25.	What are the advantages of CMOS logic and explain CMOS NAND and NOR gates with the help of a neat circuit diagram.