Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	PULSE AND WAVE SHAPING CIRCUITS			      	     Time : 3 hours

Subject Code:	11EC210/EC289									       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Justify that “coupling capacitor in RC high pass filter is called as blocking capacitor”.

2.	A square wave of peak-to-peak amplitude of 20 V is passed through a good RC differentiator. 	The peak to peak amplitude of the output is ____________.

3.	For unsymmetrical triggering in bistable multivibrator, how many trigger sources are required?

4.	Define stable state.

5.	Why monostable multivibrator is called as a univibrator?

6.	Write some applications of astable multivibrator.

7.	What is flyback time?

8.	Define displacement error in voltage time base generator.

9.	What is sampling gate?

10.	What is blocking oscillator?

PART – B  (5 x 3 = 15 MARKS)

11.	What is meant by linear waveshaping?

12.	What is loading effect in bistable multivibrator? How can you eliminate this problem?

13.	Mention any three advantages of emitter-coupled astable multivibrator over the collector-	coupled astable multivibrator.

14.	What is linear time-base generator?

15.	List out any three applications of blocking oscillator.

PART – C  (5 x 15 = 75 MARKS)

16.	Mathematically obtain the response of a low-pass filter for a square wave input. Sketch the related graphs clearly.

(OR)

17.	a.	Sketch and describe the working of the following clipping circuits: 					  (10)

(i) Series positive clipper.

(ii) Parallel negative clipper.

b.	State clamping theorem and derive its mathematical expression.					    (5)

18.	a.	With neat circuit diagram, explain the operation of fixed bias bistable multivibrator.	  (10)

b.	Write short notes on commutating capacitor.										    (5)

(OR)

19.	Elaborate the working principle of Schmitt Trigger with neat circuit diagram.  Also mention the applications of Schmitt trigger.

20.	a.	Explain the operation of collector coupled monostable multivibrator with neat circuit 	diagram.																		    (8)

b.	Derive the gate width expression of collector coupled monostable multivibrator when the first transistor Q1 is in saturation region.											    (7)

[P.T.O]

(OR)

21.	With the help of neat circuit diagram and waveform, explain the operation of free running multivibrator and derive the time period of its output waveform.

22.	Obtain an expression for the slope error es in exponential charging of capacitor in terms of the power supply voltage V.

(OR)

23.	Discuss the principles that are common to both Miller sweep and bootstrap sweep approaches commencing from the related simple RC circuit.  How does the choice of the location of the ground influence the description of the circuit?  Obtain the Miller sweep circuit and the bootstrap sweep circuit individually.

24.	Describe the operation of a triggered transistor blocking oscillator with base timing.  Make use of neat sketches of circuits and waveforms.  Obtain an expression for the pulse width tp.

(OR)

25.	Sketch a simple diode unidirectional sampling gate and explain how it functions as a coincidence gate.