Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	TRANSMISSION LINES AND WAVE GUIDES		      	     Time : 3 hours

Subject Code:	11EC212/EC280					 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Name any two types of transmission lines.

2.	What are the secondary constants of a transmission line?

3.	The condition for a distortionless line is____________.

4.	Mention any two applications of smith chart.

5.	Write down the dominant mode for TE wave in a parallel plate.

6.	Parallel plate wave guide consists of two conducting sheets separated by a__________.

7.	What is TEM wave or principal wave?

8.	The non propagating mode is called as __________.

9.	Which waveguide is easy to manufacture?

10.	The main disadvantage of circular waveguide is that its cross section is larger than that of rectangular waveguide for carrying the same signal. (True/False)

PART – B  (5 x 3 = 15 MARKS)

11.	Define standing waves and standing wave ratio.

12.	What are the applications of Quarter wave transformer?

13.	Show that for a TE10 mode, a frequency of 10 GHz will pass through a parallel plate of distance a = 10cm is inserted inside the guide.

14.	Draw a neat sketch showing the variation of attenuation with frequency of TE & TM waves in a waveguide.

15.	Define Quality factor of a resonator.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Derive the standard transmission line equation.									  (11)

b.	At 8 MHz the characteristic impedance of a transmission line is (40–j2)Ω and the propagation constant is (0.01+j0.18) per meter. Find the primary constants.			    (4)

(OR)

17.	a.	Derive the equation for α and β in a distortionless line.							    (6)

b.	Derive Campbell’s equation for the analysis of a loaded line.						    (9)

18.	Determine the input impedance of transmission line of electrical length 28o with terminated load of   ZR/Z0 =2.6+j1. Write down all the calculations and plot it on a Smith Chart.

(OR)

19.	Explain single stub matching on a transmission line and derive the expression and the length of the stub used for matching on a line.

20.	Discuss the transmission of TM waves between parallel perfectly conducting planes with necessary expressions for the field components.

[P.T.O]

(OR)

21.	Discuss the transmission of TE waves between parallel perfectly conducting planes with necessary expressions for the field components.

22.	a.	Derive the Wave impedances for TE and TM waves in rectangular waveguide.		    (6)

b.	A standard air filled X band rectangular waveguide has dimensions 7 x 3.5cm operates in dominant TE10 mode. Find the

- cut-off frequency

- Determine phase velocity at 3.5GHz

- Determine guide wavelength at a frequency of 3.5GHz.						    (9)

(OR)

23.	Derive the field equation for TE waves in a rectangular waveguide. Draw the field configuration for various lower order modes in rectangular guide.

24.	a.	Derive the characteristic equations of TM waves in circular waveguides.			  (12)

b.	Enumerate the disadvantages of circular waveguides.								    (3)

(OR)

25.	Derive the equation for Q factor of rectangular cavity resonator for TE101 mode.