Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	Electronic Devices/ Electron Devices     	Time : 3 hours

Subject Code:	11EE203/ 12EE203/ EE247						Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.   The cut in Voltage of a Si Diode is

- 0.7 v		b.	0.1v		c.	0.01v

2.	State any one application of Zener Diode.

3.	The intersection of DC load line and the characteristics graph is called ______ point.

4.	FET is a voltage controlled device. (True or False).

5.	State any one application of FET.

6.	Expand the term MOSFET.

7.	BJT is a _______ controlled device.

8.	State any one step in IC fabrication Process.

9.	What is meant by Ingot?

10.	Define Doping.

PART – B  (5 x 3 = 15 MARKS)

11.	Describe the concept of positive clamper with diagram.

12.	Draw the DC load line of a transistor with Q point.

13.	Describe the operating regions of a JFET?

14.	How MOSFET can be operated as a Switch?

15.	Write short notes on IC packaging.

PART – C  (5 x 15 = 75 MARKS)

16.	Discuss in detail the operation of PN junction diode with neat circuit diagrams.

(OR)

17.	Discuss in detail the operation of Zener diode with neat sketch for forward bias and reverse bias.

18.	Explain in detail the operation of BJT in Common emitter configuration with respect to input impedance, output impedance and also justify why it is used as an amplifier.

(OR)

19.	a. How the transistor can be working as a switch? Explain.	 						  (5)

b. How can we increase the stability of a BJT operated in forward active mode? Explain with

diagram.																			(10)

20.	Explain the “gate Biasing” of JFET.

(OR)

21.	With proper sketch explain the construction and working of a JFET.

22.	Explain in detail the operation of enhancement mode MOSFET and sketch the drain current

characteristics.

(OR)

23.	Explain in detail the operations of depletion mode MOSFET and also discuss the advantages between enhancement mode and depletion mode.

24.	a. Define epitaxial growth. Explain with diagrammatic representation.					(10)

b. Write short notes on Oxidation.													  (5)

(OR)

25.	Explain the process involved in Photolithography with its diagram.