Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	ELECTROMAGNETIC FIELDS						      	     Time : 3 hours

Subject Code:	11EE206/12EE202/EE248		 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	ax×ay⋅az=_______.

2.	Define ‘irrotational field’.

3.	Coulomb’s force on unit positive charge is called ____________.

4.	Write Laplace’s equation.

5.	What is the source for static magnetic field?

6.	Write the expression for force between two current carrying conductors.

7.	EMF induced in a transformer is called ____________.

8.	Mention any two applications of eddy current.

9.	EM wave can travel in a perfect dielectric medium without loss in strength. True/False

10.	Write the expression for reflection coefficient.

PART – B  (5 x 3 = 15 MARKS)

11.	State divergence theorem.

12.	Define electric dipole and polarization. Mention one practical application related to electric dipole.

13.	A 500-turn rectangular coil of 40 cm by 10 cm with current 2.5 A, is in a uniform magnetic 	field of 0.15 Tesla. Find the magnetic moment and the maximum torque.

14.	Derive the expression to relate conduction current density and displacement current density.

15.	Define skin effect. Mention one practical application of it.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Write the expressions for differential length, differential surface and differential volume of 	a spherical coordinate system.

b.	Transform the following vector to cylindrical coordinates at the point specified.

B=2.5ax−4ay+az at Q(1,2,0.5)

(OR)

17.	a.	Define the gradient of a scalar field. Mention its properties.						    (7)	b.	Determine the divergence of the vector field.

T=1r2cosθ ar+rsinθcosϕaθ+cosθaϕ										    (8)

18.	Derive the expression for Electric field Intensity due to an Infinite line Charge.

(OR)

19.	a.	Deduce an expression for the capacitance of a parallel plate capacitor.				    (8)

b.	A parallel plate capacitor has plate area of 10 mm2, separation of 0.1 mm with a dielectric for which ϵr= 950 and a field of 106 V/m. Calculate C and V. 						    (7)

20.	State Ampere’s law. Use it for finding the magnetic field due to co-axial cable.

(OR)

[P.T.O]

21.	Obtain the boundary conditions at the interface of two magnetic materials having permeability μ1 and μ2.

22.	Derive the expression for induced emf in a Faraday’s disc generator.

(OR)

23.	Derive Maxwell’s equation in integral and differential forms.

24.	Derive the expression for wave parameters of a good conductor.

(OR)

25.	State Poynting’s Theorem and derive its mathematical form.