Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	DC MACHINES AND TRANSFORMERS		                  Time : 3 hours

Subject Code:	11EE207 / 12EE207 / EE250						     Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is the law behind the generation of emf in a DC generator?

2.	State two functions of yoke of a DC generator.

3.	What is the reason for the no load characteristics of a self-excited DC generator not starting from the origin of the graph?

4.	State Fleming left hand rule.

5.	What are the applications of DC compound motors?

6.	Write the merit of Hopkinson’s Test.

7.	What is all day efficiency?

8.	What are the different types of transformers based on construction?

9.	What may be the reason for a low voltage in any one phase of the secondary, when other two phase has the rated voltage of the distribution transformer?

10.	Which connection of a three phase transformer is best suitable for a three phase 4 wire system?

PART – B  (5 x 3 = 15 MARKS)

11.	What is armature reaction?

12.	Why series motor has to be started with load?

13.	Why indirect methods of prediction of efficiency are necessary for any motor or generator?

14.	Draw the vector diagram of a transformer on no load.

15.	State the conditions for a parallel operation of transformers.

PART – C  (5 x 15 = 75 MARKS)

16.	a. 	A short-shunt compound generator delivers a load current of 25A at 220 V and has 	armature, series field and shunt field resistance of 1, 0.5 and 200 respectively. 	Calculate the induced emf and the armature current. Allow 0.5 V per brush drop.		    (8)

b. 	Derive the emf equation for a DC generator.										    (7)

(OR)

17.	With necessary diagrams explain the no-load and load characteristics of a self excited DC Generator.

18.	a. 	Explain the principle of operation of a DC motor.							           (7)

b. 	Write short notes on the starting methods of a DC shunt motor.				           (8)

(OR)

19.	A 200 V, 14.92 KW dc shunt motor when tested by the Swinburne method gave the following results:

Running light – armature current was 6.5 A and field current 2.2 A.

With the armature locked – the current was 70 A when a potential difference of 3V was applied to the brushes. Estimate the efficiency of the motor when working under full load condition.

[P.T.O]

20.	Derive the equivalent circuit of a single phase transformer and obtain the expression for the supply voltage in terms of impedances and current.

(OR)

21.	With necessary expressions draw the vector diagram for the practical single phase transformer for leading, lagging and unity power factor conditions.

22.	a.	Derive the condition for the maximum efficiency of a transformer.					    (7)

b.	Derive the voltage equation for a single phase transformer.						    (8)

(OR)

23.	A 20 KVA, 2200/220V, 50 Hz distribution transformer is tested for efficiency and regulation as follows:

O.C Test : 220V    4.2 A,   148W

S.C Test : 86V       10.5 A, 360W

Determine (a) core loss (b) equivalent resistance referred to primary (c) equivalent resistance referred to secondary (d) equivalent reactance referred to primary (e) equivalent reactance referred to secondary (f) regulation of transformer at 0.8 p.f lagging current (g) efficiency at full load and half load at 0.8 p.f lagging current.

24.	Write short notes on the following three phase transformer connections

a.	Star/star connection.															    (5)

b.	Delta/delta connection.														    (5)

c.	Delta/star connection.															    (5)

(OR)

25.	a.	Discuss on instrument transformers.											    (7)

b.	With necessary diagrams, explain the construction of a three phase transformer.		    (8)