Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	INDUCTION AND SYNCHRONOUS MACHINES	      	     Time : 3 hours

Subject Code:	11EE209/12EE209/EE251		 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	When induction motor is at rest, what is the value of slip?

2.	The magnitude of induced emf in the rotor also reduces by _______ times the magnitude of induced emf at standstill condition.

3.	Name the type of motor used in Ceiling fans.

4.	How can we reverse the rotation of 1-phase, split-phase motor?

5.	At what speed, a 6 pole alternator is to be driven to obtain the emf of frequency 50 Hz.

6.	What is the main purpose of parallel operation?

7.	Synchronous motors operate on any desired power factor. (True/False)

8.	State synchronous motor applications.

9.	Which test is conducted to find the values of Xd and Xq of an alternator?

10.	Name the person who developed the two reaction theory.

PART – B  (5 x 3 = 15 MARKS)

11.	Why is skewing done in squirrel cage induction motor?

12.	Draw the equivalent circuit of a single phase induction motor.

13.	Illustrate voltage regulation of Alternator and also give possible reason for voltage dip.

14.	Draw the power Vs power angle diagram of Synchronous Machines.

15.	What is meant by hunting in synchronous machines? How can it be suppressed?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Explain with neat sketch any two types of starters used in three phase induction motor.(10)

b.	List out various losses in three phase induction motor.                      				    (5)

(OR)

17.	Explain the different speed control methods of phase wound & cage induction motor.

18.	Explain using suitable diagrams:

a.	Split-phase induction motor	b.	Capacitor start induction motor

(OR)

19.	Using double filed revolving theory; explain why a single phase induction motor is not self starting.

20.	Explain in detail, EMF & MMF method of predetermining the voltage regulation of Alternator.

(OR)

21.	a.	Derive the induced emf equation of an alternator.									    (7)

b.	A 3 phase, 16 pole alternator has the following data.  Number of slots = 144, 	Conductors/slot = 4 (conductors of each phase are connected in series), Coil span = 	150electrical degrees, Speed of the alternator = 375 rpm and Flux per pole = 50 	mWb. 	Calculate the phase and line voltages.											    (8)

[P.T.O]

22.	What do you mean by synchronizing of alternators? State the conditions and then explain any two method of synchronizing.

(OR)

23.	Explain the terms direct and quadrature axis synchronous reactance’s and the method of determining them in laboratory.

24.	Explain the principle of operation and methods of starting of Synchronous motors.

(OR)

25.	Explain how V and inverter V curves are drawn, using suitable experimental setup.