Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	ELECTRICAL MACHINE DESIGN					      	     Time : 3 hours

Subject Code:	11EE211/12EE211/EE252						       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What are the main dimensions of dc machine?

2.	Define leakage co-efficient.

3.	The maximum current that flow through each brush of the dc machine is ___________.

4.	Which type of winding is used for high current, low voltage machines?

5.	What are the different types of core used for transformer?

6.	Why low voltage winding of the transformer is kept near to the core?

7.	Write the equation for the diameter of rotor of induction machine in terms of stator core Diameter.

8.	Which type of rotor slots are generally used in squirrel cage induction motors?

9.	What are the two types of poles used in salient pole synchronous machines?

10.	What is runaway speed?

PART – B  (5 x 3 = 15 MARKS)

11.	Define specific electric and magnetic loadings.

12.	Mention the advantages of having larger no. of poles in DC Machine.

13.	What are the uses of cooling tube in transformer?

14.	List out the effects of having larger air gap length for induction machine.

15.	Define short circuit ratio. What are the significances of SCR?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Discuss how the specific magnetic loadings will be chosen for rotating machine?	 	    (7)

b.	A 350kW, 500V, 450 rpm, 6 pole dc generator is built with an armature diameter of 0.87m and core length of 0.32m.The lap wound armature has 660 conductors. Calculate the specific electric and magnetic loadings.											    (8)

(OR)

17.	a.	Calculate the mmf required for air gap of a dc machine with an axial length of 20 cm (no 	ducts) and pole arc of 18 cm. The slot pitch is 27 mm, slot opening is 12 mm, air gap 	length is 6mm and useful flux is 25 mWb. Take carter’s co-efficient for slots as 0.3.     (8)

b.	Discuss various methods to determine mmf required for teeth of an Electric Machine.	    (7)

18.	a.	Derive the output equation of a D.C. machine and point out its salient features. 		    (6)

b.	A 20HP, 400 V, 1000 rpm DC motor is designed to have a square pole face. The specific magnetic loading and specific electric loadings are 0.37 Wb/m2 and 16000 ac/m respectively. Find the number of poles, main dimensions of the machine. Assume full load efficiency= 0.9 and pole arc to pole pitch ratio is 0.7.                                    			    (9)

[P.T.O]

(OR)

19.	A shunt filed coil has to develop an mmf of 9000AT. The voltage drop in the coil is 40V, resistivity of the round wire is 0.021 / m – mm2, the depth of winding is 35mm and length of mean turn is 1.4m. Design a coil so that the power dissipated is 700 W/m2 of the total coil surface. Take diameter of the insulated wire 0.2mm greater than that of bare wire.

20.	Determine the dimensions of the core, the number of turns, the cross sectional area of conductors in primary and secondary windings of a 100kVA, 2200/480V, single phase core type transformer, to operate at a frequency of 50Hz by assuming the following data.  Approximate volt per turn = 7.5Volts. Maximum flux density = 1.2wb/mm2, ratio of net core area to area of circumscribing circle is 0.6.  Ratio of height to width of window is 2.  Window space factor = 0.28.  Current density = 2.5A/mm2.

(OR)

21.	250 kVA, 6600/400V, 3-phase core type transformer has a total loss of 4800W on full load. The transformer tank is 1.25m in height and 1m×0.5m in plan. Design a suitable scheme for cooling tubes if the average temperature rise is to be limited to 350C. The diameter of the tube is 50mm and is spaced 75mm from each other. The average height of the tube is 1.05m.

22.	Determine the main dimensions, stator slots and number of conductors for a 11 kW, 400V, 3phase, 4 pole, 1425 rpm, ∆ connected induction machine. Average gap density 0.45 Wb/mm2.  Ampere conductor per meter 23000, full load efficiency 0.85, power factor 0.88, ratio ofLτ=1.

(OR)

23.	Design a cage rotor for a 40HP, 3 phase, 400V, 6 pole, Delta connected induction Motor having a full load efficiency of 87% and full load power factor of 0.85. Take D=33cm and L=17 cm, stator slots 54, Conductors/slot=14. Assume suitably the missing data if any.

24.	For a 250kVA, 1100V, 50Hz, 500 RPM, 12 pole, three phase, star connected alternator, determine the main dimensions, number of stator conductors, number of stator slots and cross section of stator conductors. Assuming average gap density as 0.6Wb/m2 and specific electric loading of 30000Ampere conductors/m, Lτ=1.5.

(OR)

25.	Determine the main dimensions of a 1000 kVA, 50 Hz, 3 phase, 375 rpm alternator. The average air gap flux density is 0.55 Wb/m2 and ampere conductors per meter is 28000. Given that L/τ must be between 1 to 5. The maximum permissible peripheral speed is 50 m/sec. The runaway speed is 1.8 times synchronous speed.