Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	GENERATION, TRANSMISSION AND DISTRIBUTION

Time : 3 hours

Subject Code:	11EE215/12EE215/EE253						       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The ___________ of nuclear power plant slows down the neutrons before they bombard the fuel rods.

2.	What information does the area under the load curve or load duration curve provide?

3.	Define resistance of the transmission line.

4.	The choice of proper transmission voltage is essentially a question of __________.

5.	What is Corona?

6. 	Define Sag.

7.	The metallic sheath is provided over the insulation to protect the cable from _______.

8.	If the length of a cable is doubled, its capacitance is _______.

9.	What is a feeder?

10.	Write the advantages of ring main system.

PART – B  (5 x 3 = 15 MARKS)

11.	List the different types of nonconventional energy sources.

12.	What is proximity effect?

13.	List out the desirable properties of an Insulator.

14.	Why are single core cables not provided with armoring in AC transmission systems?

15.	What is a distributor?

PART – C  (5 x 15 = 75 MARKS)

16.	With neat sketches, explain the layout and construction of a Hydro power station.

(OR)

17.	Discuss on the importance of Non-Conventional Energy sources in today’s scenario and also explain any one of them.

18.	Calculate the capacitance of a 100 km long 3-phase, 50 Hz overhead  transmission line  consisting of 3 conductors, each of diameter 2 cm and spaced 2.5 m at the corners of an equilateral triangle.

(OR)

19.	Derive the equation to get Inductance of symmetrical 3 phase over head transmission line.

20.	a.	Discuss the advantages and disadvantages of:	i.	pin-type insulators ii.	suspension 			type insulators.											 					(5)

b.	Define String Efficiency. Show that in a string of suspension insulators, the disc nearest to

the conductor has the highest voltage across it.  									(10)

(OR)

21. 	a.	What are the factors which affect corona? Discuss the advantages and disadvantages of      		Corona.																		(5)

b.	Deduce an approximate expression for sag in overhead lines when:			 		(10)

i.	supports are at equal levels       ii.	supports are at unequal levels.

[P.T.O]

22.	Compare overhead lines with underground cables.

(OR)

23.	Derive an expression for capacitance in three core cables.

24.	Explain the following connection schemes of distribution system.

a.	Radial system			b.	Ring main system	                		        		(8+7)

(OR)

25.	A 2-wire d.c. distributor cable AB is 2 km long and supplies load of 100 A,150 A, 200 A and 50A situated 500 m, 1000 m,1600 m, and 2000 m from the feeding point A. Each conductor has a resistance of 0.01Ω per 1000 m. Calculate potential difference at each point if a potential difference of 300 V is maintained at point A.