Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	POWER SYSTEM ANALYSIS						      	       Time: 3 hours

Subject Code:	11EE216/12EE216/EE255		 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Given: Base voltage = 22,000V and Base power = 3MVA.What is the value of base impedance?

2.	1+a+a2+a3= ________.

3.	In type 1 modification of Z-bus building algorithm, off diagonal elements are always _______.

4.	In which fault zero sequence current will not flow?

5.	Mention the different types of bus in load flow studies.

6.	What is the size of the Jacobian matrix for a 15 bus power system with 3 voltage controlled buses?

7.	Mention anyone method used to do the unit commitment problem.

8.	Write down the Power balance equations.

9.	A system is said to be stable, if and only if dp/dδ is positive. (True/False)

10.	List the methods of solving swing equation.

PART – B  (5 x 3 = 15 MARKS)

11.	Write short notes on Fortescue’s theorem.

12.	How symmetrical faults are analyzed?

13.	Discriminate Gauss Seidal and Newton Raphson method in the load flow study.

14.	What are the different types of constraints used in economic dispatch problem?

15.	How can we classify stability studies?

PART – C  (5 x 15 = 75 MARKS)

16.	Illustrate in detail the need for system analysis in planning and operation of power system.

(OR)

17.	Two generators are connected in parallel to the same bus and have = 10%,

Generator (1) is rated 2,500 KVA, 2.4 KV.

Generator (2) is rated 5,000 KVA, 2.4 KV.

Find the p.u reactance of each generator on a 15000 KVA, 2.4 KV base. What is the p.u reactance of a single generator equivalent to the two generators in parallel on the same base?

18.	a.	Form Y-bus using Direct Inspection method for the following network.		       		    (6)

| S. No | #### Bus code | Mutual admittance | Bus code | Line charging admittance |
| --- | --- | --- | --- | --- |
| 1 2 3 4 | 1-2 1-3 2-3 3-4 | 0.1+0.2j 0.1+0.2j 0.1+0.2j 0.1+0.2j | 1 2 3 4 | 0.1j 0.2j 0.3j 0.4j |

b.	Derive the fault current expression for single line to ground fault occurs in a power system 	network.																		    (9)

[P.T.O]

(OR)

19.	Explain briefly about the formation of bus impedance matrix with an illustration.

20.	Derive Gauss-Seidal algorithm for load flow studies. Give the flow chart and step by step procedure to implement the algorithm.

(OR)

21.	Explain the step by step computational procedure for the Newton Raphson method in load flow studies with appropriate expression.

22.	A power plant has three units with the following fuel cost equations:

The demand is 800MW. Neglecting Line losses and generator limits. Find the optimal scheduling and also fuel cost.

(OR)

23.	a.	Explain the various constraints in unit commitment problem with suitable examples.	    (7)

b.	Give the elementary idea of optimal load scheduling of Hydro - Thermal plants.		    (8)

24.	Explain the Runge – kutta method of analyzing multi machine power system for stability with a neat flow chart.

(OR)

25.	Derive the expression for swing equation in transient stability analysis.