Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	CONTROL SYSTEMS								      	     Time : 3 hours

Subject Code:	11EE219/12EE219/EE259			 			      Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is a signal flow graph?

2.	Define System.

3.	Give the Laplace transform for unit step signal.

4.	Define steady state error.

5.	Define phase cross over frequency.

6.	The frequency at which the gain Gjω is equal to one is called _________.

7.	What is characteristic equation?

8.	What is centroid?

9.	Write down the state model equation of a continuous time invariant system.

10.	List out any two properties of state transition matrix.

PART – B  (5 x 3 = 15 MARKS)

11.	Find the overall transfer function using block diagram reduction technique.

G1

G2

H1

R(S)

C(S)

12.	The open-loop transfer function of a unity feedback servo system is given by Gs=10s0.1s+1.  Evaluate the static error constants for the system.

13.	Find the resonant peak Mr and resonant frequency ωrof a unity feedback system with 	Gs=100ss+10.

14.	What is Nyquist stability criterion?

15.	Obtain the characteristic equation of the matrix	A=010302−12−7−6

[P.T.O]

PART – C  (5 x 15 = 75 MARKS)

16.	Find the overall transfer function of the system whose signal flow graph is shown as below.

(OR)

17.	Derive the transfer function X1(s)/F(s) and X2(s)/F(s) for the given mechanical system.

M1

M2

x1

f(t)

x2

K1

K2

B1

18.	Determine time response specifications for a unit step input to a unity feedback system havingGs=144ss+2.

(OR)

19.	Draw the block diagram of a control system with proportional plus derivative controller and discuss its effects on the time response.

20.	Construct Bode plot and find stability for a unity feedback system having  Gs=2000ss+1s+100. Determine GM, PM, ωgcandωpc.

(OR)

21.	Draw the polar plot of the functionGs=8s+1s+2.

22.	The characteristic equation of a system is given by s5 + 5s4 + 11s3 + 23s2 + 28s + 12 = 0. Determine the stability using Routh-Hurwitz criteria.

(OR)

23.	Sketch the root locus of the unity feedback system whose open loop transfer function is given by Gs=Kss2+6s+25; and K varies from 0 to ∞.

24.	Find the state equation and output equation for the system given by

YsUs=s3+5s2+6s+1s3+4s2+3s+3

(OR)

25.	Determine the transfer function of the system represented by the state model

x=−3−120x+10u ;         y=10x