Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	ELECTRICAL AND ELECTRONIC INSTRUMENTATION													      	     								      Time : 3 hours

Subject Code:	11EI201								 			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What are the types of damping?

2.	State any two errors associated with moving iron instruments.

3.	Mention the application of Hay’s bridge.

4.	Name the two factors causing errors in bridge circuits.

5.	What is the main factor to be considered for the selection of resistors used as multipliers?

6.	Expand the term ‘RMS’.

7.	List the applications of CRO.

8.	List the two storage techniques used in storage CRT’s.

9.	Narrate any one advantage of digital transducer.

10.	State any two functions of a data logger.

PART – B  (5 x 3 = 15 MARKS)

11.	Write short notes on braking systems in an energy meter.

12.	List the limitations of wheat stone bridge.

13.	Mention the applications of spectrum analyzer.

14.	What are the main components of CRT?

15.	Explain the operating principle of an X-Y plotter.

PART – C  (5 x 15 = 75 MARKS)

16.	List the advantages and possible errors in Permanent magnet Moving Coil mechanism.

(OR)

17.	Draw the circuit of D’ Arsonval galvanometer and explain its working.

18.	Describe the Maxwell bridge method of measuring inductance with a neat diagram.

(OR)

19.	Derive the expression for sensitivity of a Wheatstone bridge with equal arms and explain its applications.

20.	Explain the harmonic distortion analyzer using a block diagram.

(OR)

21.	The schematic diagram of Vector impedance meter and discuss its operation.

22.	Draw the block diagram of CRO and explain.

(OR)

23.	With a neat sketch explain the vertical deflection system of a CRO.

24.	With relevant schematic explain the digital method used for measuring frequency.

(OR)

25.	Explain the operation of a data logger with neat sketches.