Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	PROCESS ENGINEERING CALCULATIONS		      	     Time : 3 hours

Subject Code:	11FP202								 		      Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The dimension for viscosity is ___________.

2.	One mole of any substance will contain ___________ no of molecules/atoms.

3.	Material balance is based on law of ___________.

4.	In wet bulb temperature the bulb is covered with ___________.

5.	Raoult’s law is applicable only for ___________ solutions.

6.	Absorption is a mass transfer operation which involves ___________ and ___________.

7.	The final unit operation in most of the process industries is ___________.

8.	Dialysis is an example for ___________ separation process.

9.	The heat which can be measured by using thermometer is called ___________ heat.

10.	The enthalpy associated with the mixing of two liquids is called ___________.

PART – B  (5 x 3 = 15 MARKS)

11.	What are the various methods of expressing the composition of mixtures?

12.	Under what circumstances a real gas will show ideal behavior and why?

13.	Distillation is based on what principle and what is relative volatility?

14.	What is the role of entrainment separator in evaporator?

15.	What is standard heat of formation?

PART – C  (5 x 15 = 75 MARKS)

16.	What are fundamental and derived units? Give example. Discuss about the viscosity, specific gravity, specific volume and density for the case of liquid and gas.

(OR)

17.	A solution of potassium hydroxide (KOH) in water contains 250gm of KOH per liter of solution a 27°C. The densities of solution and of water at 27°C are 1.2 and 1.0gm/cc respectively. Calculate

a.	Composition in weight percent.

b.	Composition in volume percent.

18.	A natural gas having the composition CH4 – 94%, C2H6 – 3% and N2 – 3% is piped from the well at 25°C and 3.0 atm. pressures. Assuming that the ideal gas law is obeyed, find out:

a.	Partial pressure of N2.

b.	Volume of N2 per 100 m3 of gas.

c.	Density of gas.

(OR)

19.	What are the various equation of state? Discuss about the advantage and disadvantage.

20.	With a neat sketch explain the process flow diagram for the production of ethanol from molasses.

(OR)

[P.T.O.]

21.	Write short notes on Distillation, Absorption and extraction.

22.	What are all the factors that will affect evaporation and explain the construction and operation of single effect evaporator?

(OR)

23.	An evaporator is fed continuously with 50000 Kg/hr of a solution containing 10% NaOH, 10% NaCl and 80% water by weight. During the evaporation, water is boiled off and NaCl precipitates as crystals and removed from the remaining liquor. The concentrated liquor leaving the evaporator contains 50% NaOH, 2% NaCl and 48% water. Calculate

a.	Kg of water evaporated per hour

b.	Kg of salt precipitated per hour

c.	Kg of concentrated liquor produced per hour

24.	The following enthalpy changes are known from experiment for the reactions below, at 25°C in standard state. Calculate the standard heat of formation of propylene gas.

Reactions				           				 	 ∆H°Kcal/g mole

C3H6 (g) + H2 (g) ------------ C3H8 (g)	                             -29.6

C3H8 (g) + 5O2 (g) --------------- 3CO2 (g) +4 H2O (l)  		-530.60

(g) + ½ O2 (g)  ----------------- H2O (l)                           	- 68.30

C (graphite) + O2 (g) ----------- CO2 (g)		      			-94.05

(OR)

25.	Calculate the energy required in kcal to heat 1 kg mole of hydrogen from 20°C to 140°C

CpH2 = 6.946 – 0.196x10-3T + 0.4757x10-6T2 Kcal/Kg mole K,where T is in Kelvin.