Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	PRINCIPLES OF FLUID MECHANICS AND HEAT TRANSFER

Time : 3 hours

Subject Code:	11FP204						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define kinematic viscosity.

2.	What do you mean by turbulent flow?

3.	State the first law of thermodynamics.

4.	Define reversible process.

5.	Define dryness fraction of steam.

6.	What are the two zones of boilers?

7.	What is the driving force in heat transfer?

8.	Give examples of some materials which have high thermal conductivity.

9.	Define forced convection.

10.	Define emissivity.

PART – B  (5 x 3 = 15 MARKS)

11.	Write a note on manometer.

12.	Briefly describe the isothermal process.

13.	Write notes on properties of steam.

14.	Find the rate of heat loss through a stainless steel slab 10 cm thick which is maintained at 100 ℃ on the hot side and 30 ℃ on the cold side. The thermal conductivity of the steel is 16.37 W/m ℃.

15.	Briefly explain the basics of radiation heat transfer.

PART – C  (5 x 15 = 75 MARKS)

16.	Derive Bernoulli’s equation.

(OR)

17.	Explain the principle and working of Orifice meter with a neat sketch.

18.	Derive the expression for adiabatic process of an ideal gas.

(OR)

19.	Explain in detail about carnot cycle.

20.	Explain the Pressure-Enthalpy and Pressure-Volume diagrams.

(OR)

21.	Describe the working of vertical tube boiler with diagram.

22.	Explain the process of conduction through a flat composite wall.

(OR)

[P.T.O]

23.	Find the heat transfer rate per one metre length of circular copper pipe of 10 cm inner diameter and 12.5 cm outer diameter. The inside temperature of the pipe is 100℃ and the outside temperature of the pipe is 40℃. Also calculate the percentage error involved by using average radius in place of log mean radius. The thermal conductivity of the metal is 850 W/m℃.

24.	Derive the equation for overall heat transfer coefficient in convection heat transfer.

(OR)

25.	A dilute pineapple juice is heated in a double pipe heat exchanger from 28℃ to 75℃ by heat exchanging with hot water flowing in shell in counter current direction and is cooled from 95℃ to 85℃. Calculate the log mean temperature difference and compare with average temperature difference.