Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	ENGINEERING PROPERTIES OF FOOD MATERIALS

Time : 3 hours

Subject Code:	11FP221/12FP215								       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Screening and grading of food materials are based on ___________.

2.	Aspect ratio is used for expressing ___________.

3.	Name one instrument used in cutting shear.

4.	Define shear stress.

5.	What do you mean by thermal diffusivity?

6.	How is heat conducted in pure metals?

7.	Give an application of estimation of water activity.

8.	State Henry’s Law.

9.	Near infrared radiation can be used to measure ___________ of foods.

10.	Give the two types of reflection of electromagnetic radiation.

PART – B  (5 x 3 = 15 MARKS)

11.	The major diameter (L) and the average of the minor and major diameters (H) of barley are measured as 8.76 mm and 2.83 mm, respectively. Calculate the minimum and maximum radii of curvature for the barley.

12.	Define bulk modulus and modulus of rigidity.

13.	Write a note on Series Model of heat flow.

14.	Define terminal velocity.

15.	Write a short note on dipolar rotation.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Explain with a neat sketch how to determine the volume through Solid and liquid 	displacement method.															  (10)

b.	Write a short note on mercury porosimetry.										    (5)

(OR)

17.	Explain in detail about the following:

a.	Funnel flow.

b.	Mass flow.

c.	Expanded flow.

18.	a.	Describe with a neat sketch working of Ostwald’s viscometer.						    (8)

b.	Write down the application of rheology in food industry.							    (7)

(OR)

19.	Explain in detail about Texture Profile Analysis.

[P.T.O]

20.	a.	Describe in brief about Fourier’s law of heat conduction. 							    (6)

b.	Explain various types of thermal conductivity measurement under Heat of vaporization and 	Heat flux methods.															    (9)

(OR)

21.	a.	Estimate the speciﬁc heat of potatoes containing 85% water.

Data:

Specific heat of water = 4186.80 J/kg K

Specific heat of nonfat solids = 837.36 J/kg K									    (5)

b.	Calculate the speciﬁc heat of wild rice grain at 20◦C with the approximate composition data given in the following table

| Component | Weight % |
| --- | --- |
| Water | 8.5 |
| Carbohydrate | 75.3 |
| Protein | 14.1 |
| Fat | 0.7 |
| Ash | 1.4 |

(10)

22.	Describe with a neat sketch the Water Activity determination by Vapor Pressure Measurement.

(OR)

23.	Discuss the effects of pressure and temperature over water activity.

24.	Discuss effects of the following on Dielectric properties of foods.

a. Moisture content     b.  Temperature

(OR)

25.	Illustrate with a sketch any one method used for determination of Dielectric properties of foods.