Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	DESIGN OF HEAT EXCHANGERS AND PRESSURE VESSELS FOR FOOD PROCESSING								       	      Time : 3 hours

Subject Code:	11ME205						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Two main classifications of heat exchangers are _______ and _______.

2.	LMTD is called _______.

3.	Baffles are used to enhance _______.

4.	The heat transfer co-efficient in counter current flow is _______ than the co current flow.

5.	The tubes are arranged in the tube sheet of shell and tube heat exchanger either by _______ or_______ pitch.

6.	Two types of condensation are _______ and _______.

7.	The purpose of using agitator is to enhance _______.

8.	Heating by using steam jackets are more suitable for _______ operation.

9.	The purpose of using evaporators is to _______ the weak solution.

10.	Multiple effect evaporators are used to increase the _______.

PART – B  (5 x 3 = 15 MARKS)

11.	What is fouling factor?

12.	What is overall heat transfer coefficient?

13.	What is the difference between double pipe heat exchanger and shell and tube heat exchanger?

14.	What are recuperative and regenerative heat exchangers?

15.	What are the different methods of feeding the multiple effect evaporators?

PART – C  (5 x 15 = 75 MARKS)

16.	What are the classifications of heat exchangers? Explain the construction and operation of shell and tube heat exchanger.

(OR)

17.	Crude oil flows at the rate of 10,000 kg/hr through the inside pipe of a double pipe heat exchanger and is heated from 32°C to 90°C. The heat is supplied by a petroleum fraction initially at 230°C, flowing through the annular space. If the temperatures of the heating fluid falls down to 100°C inside the heat exchanger compare the performances of parallel and counter current exchangers with respect to the heat transfer area and fluid flow rates. Overall heat transfer co efficient is 400Kcal/hrm2°C. Specific heats of crude oil and petroleum fraction are 0.56 and 0.6 Kcal/Kg°C respectively.

18.	What is the use of baffles in heat exchangers? Explain various types in detail.

(OR)

19.	In a water to water heat exchanger with counter flow arrangement and having a heating surface of 2 m2, the inlet temperature and flow rate of heating water were 85°C and 2000 kg/hr. The flow rate and inlet temperature of the water being heated were 1500 Kg/hr and 25°C. Calculate the rate of heat transfer from the hot to cold water and give their final temperatures, if U =1200 Kcal/hrm2°C.

[P.T.O]

20.	Discuss about the Design consideration for cooling towers in detail.

(OR)

21.	What are the two types of condensation and which is preferable? Explain in detail.

22.	What is air cooled heat exchanger and what are the advantages and disadvantages?

(OR)

23.	It is desired to heat a given quantity of air from 30°C to 80°C. The air passes inside a number of tubes arranged in parallel and is heated saturated steam condensing at 130°C. Selecting 3mm i.d. tubes and a reasonable G (kg/hrm2), it is calculated that 100 tubes each 5 meter long and in parallel are required. As the actual heat transfer co efficient may be somewhat lower than that predicted by Colburn analogy, it is agreed to use a factor of safety of 1.2. This may be achieved by i) using in parallel 20% more tubes or ii) increasing tube diameter by 20%. Discuss the situation and submit your recommendations.

24.	a.	What are the factors that affect evaporation?										     (7)

b.	What is the need for multiple effect evaporators? Explain various methods of feeding the multiple effect evaporators.													     (8)

(OR)

25.	Derive the material and energy balance equation for the triple effect evaporator starting from law of conservation of mass and energy.