Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	DIGITAL ELECTRONICS							      	     Time : 3 hours

Subject Code:	11MT210/ 12MT211/ MT227						       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Convert (673)8 to binary.

2.	Find the two’s complement of (0110)2

3.	Draw the truth-table of a XNOR gate.

4.	What are the applications of decoder?

5.	What is a clocked sequential circuit?

6.	Draw the excitation table of D flip-flop.

7.	What is edge triggering?

8.	What is an excitation table?

9.	Which is the fastest logic family?

10.	What is tri-state output?

PART – B  (5 x 3 = 15 MARKS)

- 11.	State De Morgans theorem. Find the complement of F=xy’ +x’y

12.	Implement Y = (0, 2, 4, 6) using logic gates.

13.	Derive the characteristic equation of SR flip-flop.

14.	What is an essential hazard?

15.	Define totem pole output. What is its advantage?

PART – C  (5 x 15 = 75 MARKS)

16.	Simplify the Boolean function F(A,B,C,D,E) =(0, 2, 4, 6, 9, 13, 21, 23, 25, 29, 31).

(OR)

17. 	Given the following Boolean function F = A’C +A’B +AB’C +BC.

a.	Express it in sum of minterms

b.	Find the minimal sum of products expression.								       (7+8)

18.	Design a Full subtractor using  a) two half-subtractors and b)NAND gates.				(7+8)

(OR)

19.	Design a 4 – bit Magnitude comparator.

20.	With a brief note on need for master-slave flip-flops, explain a master slave JK flip-flop in detail.

(OR)

21.	Explain the operation of a 4 bit Serial – in Parallel out shift register with a timing diagram.

22.	Explain the Mealy and Moore Models of sequential circuit.

(OR)

23.	Design a sequence detector that produces an output ‘1’ whenever the non-overlapping sequence

1011 is detected.

24.	Explain the operation of 	a)TTL NAND gate with Totem-pole output 	b)Resistor Transistor logic   c)Emitter coupled logic

(OR)

25.	Design a BCD to Excess-3 code converter using PAL.