Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	LINEAR INTEGRATED CIRCUITS					      	     Time : 3 hours

Subject Code:	11MT211										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	__________ in electronics is the method of establishing predetermined voltages or currents at various points of an electronic circuit for the purpose of establishing proper operating conditions in electronic components.

2.	__________ is the maximum voltage change per unit time in a node of a circuit, due to limited current sink or source.

3.	A _________ is an electronic circuit used to implement a variety of simple multi-state systems such as oscillators, timers and flip-flops.

4.	__________ is a bistable circuit in which the output increases to a steady maximum when the input rises above a certain threshold, and decreases almost to zero when the input voltage falls below another threshold.

5.	A __________ is an oscillator with an output signal whose output can be varied over a range, which is controlled by the input DC voltage.

6.	A ___________ is an electronic system for generating any of a range of frequencies from a single fixed time base or oscillator.

7.	A ____________ is a type of analog-to-digital converter that converts a continuous analog waveform into a discrete digital representation via a binary search through all possible quantization levels before finally converging upon a digital output for each conversion.

8.	____________ is also used to transfer high bit-count low frequency digital signals into lower bit-count higher frequency digital signals as part of the process to convert digital signals into analog.

9.	A _________ is an electronic amplifier which includes band pass filtering components within the amplifier circuitry.

10.	An _________ is an electronic component that interconnects two separate electrical circuits by means of a light sensitive optical interface.

PART – B  (5 x 3 = 15 MARKS)

11.	What is frequency compensation and why it is important?

12.	Mention the significance of Sine Wave Oscillator.

13.	State the advantages of Compander ICs.

14.	What are the various types of Digital to Analog Converter Techniques?

15.	Write the advantages of Opto-couplers.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain in detail about the analysis of difference amplifiers with active loads with suitable circuit diagrams and mathematical analysis.

(OR)

17.	Describe the concept of frequency compensation in detail. Also explain slew rate and the various techniques to improve it.

[P.T.O]

18.	Illustrate about a) Inverting and Non inverting Amplifiers b) Differentiator and Integrator in detail.

(OR)

19.	Explain in detail about the a) Precision Rectifier b) Log and Antilog Amplifiers with suitable diagrams.

20.	Analyze the Gilbert cell with suitable mathematical analysis and circuit diagrams.

(OR)

21.	With a neat diagram explain the working principle of frequency synthesizer and compander ICs.

22.	Elucidate any two types of Digital to Analog Converters in detail.

(OR)

23.	Brief on the working principle of

a.	Delta Sigma Modulation

b.	Voltage to Time Converters.

24.	Why voltage regulators are significant? Explain the linear and switched mode types of voltage regulators.

(OR)

25.	With a block diagram, explain the concept of a) Power amplifiers b) Isolation amplifiers.