Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	ELECTRONIC COMMUNICATION SYSTEMS		      	     Time : 3 hours

Subject Code:	11MT216								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Give an expression for energy signal and power signal.

2.	Define Signal to Noise Ratio.

3.	Define Amplitude Modulation.

4.	Write any two advantages of double-conversion receiver over single-conversion receivers.

5.	Write the applications of FM.

6.	Define Deviation ratio with expression.

7.	PCM Stands for __________.

8.	Write the advantages of Time Division Multiplexing.

9.	Differentiate Bit rate and Baud rate.

10.	A __________ is a code chosen for use within a communications system for transmitting a digital signal down a line.

PART – B  (5 x 3 = 15 MARKS)

11.	The parallel tuned circuit at the input of a radio receiver is tuned to resonate at 120 MHz by a capacitance of 25pF.The Q factor of the circuit is 30.The channel bandwidth of the receiver is limited to 10 KHz by the audio sections. Calculate the effective noise voltage appearing at the input at room temperature.

12.	Define adjacent channel selectivity.

13.	What is pre-emphasis and de-emphasis? Where is it used?

14.	Write a short note on noise in FM.

15.	Briefly explain the blocks in Differential pulse code modulation transmitter and receiver.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain in brief about the different sources of noise.

(OR)

17.	a.	Explain about thermal noise modeling in detail.									    (8)

b.	What is the relationship between noise temperature and noise factor. Give the expression.

(7)

18.	Elaborate different methods to generate single sideband signals.

(OR)

19.	a.	Explain in detail on Amplitude Modulator circuits.								  (10)

b.	What is meant by Super heterodyne receiver?									    (5)

20.	a.	Differentiate FM and PM.														    (5)

b.	Explain angle modulator detector.												  (10)

(OR)

21.	Using suitable mathematics, derive the output equation for an FM system. Also sketch its output waveform in both time domain and frequency domain.

[P.T.O]

22.	a.	Describe briefly, about the Pulse Amplitude modulation scheme.					  (10)

b.	Define Differential PCM with diagram.											    (5)

(OR)

23.	What is meant by delta modulation? Draw the block diagram of a DM transmitter and receiver. Explain its operation.

24.	With a neat block diagram explain about the digital communication system.

(OR)

25.	a.	Define QPSK in detail with necessary diagrams. Write its advantages over other 	modulation schemes.															  (10)

b.	Explain the concept of FSK in detail.											    (5)