Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	THERMODYNAMICS								      	     Time : 3 hours

Subject Code:	12AE202						 				       Maximum Marks: 100

#### Answer ALL questions

Use of standard steam tables is permitted

Standard data not provided in the problem may be assumed suitably

PART – A  (10 x 1 = 10 MARKS)

1.	Name 3 categories of classification of thermodynamic systems.

2.	A constant temperature process is also known as __________.

3.	In a reversible cycle, the entropy of the system __________.

4.	Define available energy in a system.

5.	What is meant by triple point?

6.	Define dryness fraction.

7.	When is a system referred as monovariant?

8.	There is __________ in temperature, when an ideal gas undergoes a Joule-Kelvin expansion.

9.	Write the time-independent Schrodinger wave equation.

10.	State the equipartition of energy.

PART – B  (5 x 3 = 15 MARKS)

11.	Define equilibrium. What are the different conditions of equilibrium ?

12.	Draw and state Clausius statement of the second law of thermodynamics.

13.	Draw a p-V-T surface and show the various equilibrium states.

14.	State and explain Gibbs phase change rule for a non-reactive system.

15.	Write a note on the wave-particle duality of light.

PART – C  (5 x 15 = 75 MARKS)

16.	Air at 1.02 bar, 22C, initially occupying a cylinder volume of 0.015 m3, is compressed reversibly and adiabatically by a piston to a pressure of 6.8 bar. Calculate a) the final temperature b) final volume and c) work done. Take  for air as 1.4 and R for air as 287 J/kg k.

(OR)

17.	At the inlet of a certain nozzle, the enthalpy of fluid passing is 2800 kJ/kg, and the velocity is 50 m/s. At the discharge end, the enthalpy is 2600 kJ/kg. The nozzle is horizontal and there is negligible heat loss from it. a) Find the velocity at the exit of the nozzle b) If the inlet area is 900 cm2 and the specific volume at the inlet is 0.187 m3/kg, find the mass flow rate c) If the specific volume at the nozzle exit is 0.498 m3/kg, find the exit area of the nozzle.

18.	a.	Explain the working of a Carnot cycle and discuss the processes with the help of a p-V 	diagram.																		  (10)

b.	What is the highest possible theoretical efficiency of a heat engine operating with a hot reservoir of furnace gases at 2100C when the cooling water available is at 15C?        (5)

(OR)

19.	An insulated cylinder of volume capacity 4 m3 contains 20 kg of Nitrogen. Paddle work is done on the gas by stirring it till the pressure in the vessel increases from 4 bar to 8 bar. Determine a) change in internal energy b) work done c) heat transferred d) change in entropy. Take Cp for Nitrogen as 1.04 kJ/kgK and Cv for Nitrogen as 0.7432 kJ/kgK.

[P.T.O]

20.	A spherical vessel of 0.9 m3 capacity contains steam at 8 bar and 0.9 dryness fraction. Steam is blown off until the pressure drops to 4 bar. The valve is then closed and the steam is allowed to cool until the pressure falls to 3 bar. Assuming that the enthalpy of steam in the vessel remains constant during blowing-off periods, determine a) mass of steam blown off b) dryness fraction of the steam in the vessel after cooling c) heat lost by steam per kg during cooling.

(OR)

21.	The following observations were taken with a separating and a throttling calorimeter arranged in series.

Water separated = 2 kg

Steam discharged from the throttling calorimeter = 20.5 kg

Temperature of steam after throttling = 110C

Initial pressure = 12 bar abs

Barometer = 760 mm of Hg

Final pressure = 5 mm of Hg.

Estimate the quality of steam supplied.

22.	Derive the Tds relations and show that

(OR)

23.	The virial equation of state of gas is given by pV = RT (1+Bp +Cp2+…..). Show that the inversion temperature of a vanderwall’s gas is twice the Boyle’s temperature.

24.	a.	Consider a cubical box of edge 10 cm, containing gaseous helium at 300 K. Evaluate the 	energy x and its corresponding nx.												  (10)

b.	Write a short note on Heisenberg’s uncertainty principle.							    (5)

(OR)

25.	a.	Write a brief note on Maxwell-Boltzmann statistics.								  (10)

b.	Calculate the number of ways of arranging seven distinguishable particles in four boxes so that N1=1, N2=2, N3=3 and N4=1.												    (5)