Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	STRENGTH OF AEROSPACE MATERIALS		      	     Time : 3 hours

Subject Code:	12AE203								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define Poisson’s Ratio.

2.	State Hooke’s law.

3.	A simply supported beam of span ‘l’, carries a point load ‘W’ at the mid-span of the beam. The maximum bending moment is given by ___________.

4.	Draw the bending moment diagram for a cantilever beam of span L subjected to uniformly distributed load  w over the entire span.

5.	Define neutral axis.

6.	The section modulus Z of a hollow rectangular section  is ___________.

7.	Write the Euler’s critical load formula for a column with one end fixed and other end free.

8.	The deflection at the centre of a simply supported beam carrying a point load at the centre is ___________.

9.	Define the terms: Principal planes and principal stresses.

10.	What is the criterion of failure for Maximum principal stress theory (or Rankine Theory)?

PART – B  (5 x 3 = 15 MARKS)

11.	A steel rod of 30 mm diameter and 5 m long is connected to two grips and the rod is maintained at 95˚C. Determine the stress exerted when the temperature falls to 30 ˚C if the ends do not yield. (α = 12 X 10-6 / ˚C and E = 2 X 105 MN/m2)

12.	A cantilever of length 2 m carries a UDL of 2KN/m run over a length of 1m measured from the fixed end.  Draw SFD for the cantilever.

13.	Write the assumptions made in the theory of simply bending.

14.	Define the terms: columns, struts and crippling load.

15.	The principal stresses at appoint in an elastic material are 100N/ mm2 (tensile), 80 N/ mm2 (tensile), and 50 N/ mm2 (compressive). If the stress at the elastic limit in simple tension is 200 N/ mm2, determine whether the failure of material will occur according to Maximum principal stress theory, if not then determine the factor of safety.

PART – C  (5 x 15 = 75 MARKS)

16.	A bar having cross-sectional area of 1000 mm2 is subjected to axial forces as shown in below figure. Find the total elongation of the bar . E (Brass) = 1.05 ×105 N/mm2 .

[P.T.O]

(OR)

17.	a.	Explain clearly the different types of stresses and strains.							    (7)

b.	Define a composite bar. How will you find the stresses and load carried by each member of a composite bar?																    (8)

18.	Draw the shear force and bending moment diagram for the given beam.

(OR)

19.	Draw the shear force and bending moment diagrams for the beam shown in Fig. Also determine the maximum bending moment and location of point of contra flexure.

20.	A rolled steel joist of I-section has the following dimensions:

flange – 250 mm wide and 24 mm thick;

web – 12 mm thick;

overall depth – 600mm

if this beam carries a uniformly distributed load of 50 kN/m run on a span of 8m, calculate the maximum stress produced due to bending.

(OR)

21.	Design a shaft to transmit 1mw of power at 300rpm. The stress in the shaft should not exceed 60mpa, and the angle of twist should not be more than 1 degree in a length of 2m. Assume Modulus of rigidity   = 75 GPa.

22.	Derive an expression for Euler’s crippling load for a long column with following end conditions

a.	When both ends of the column are hinged.

b.	When one end of the column is fixed and the other end free.						(7+8)

(OR)

23.	A beam of uniform rectangular section 200 mm wide and 300 mm deep is simply supported at its ends. It carries a uniformly distributed load of 9 kN/m run over the entire span of 5 m. If the value of E for the beam material is 1×104 N/mm2, find

a.	The slope at the supports and

b.	Maximum deflection.

24.	At point in a strained material is subjected to the stresses as shown in fig. Locate the principal planes, and evaluate the principal stresses.

40N/mm2

60

60N/mm2

60N/mm2

40N/mm2

(OR)

25.	Define and explain the following theories of failure 									(5x3)

a.	Maximum principal stress theory.

b.	Maximum principal strain theory.

c.	Maximum shear stress theory.