Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	THERMAL ENGINEERING						      	     Time : 3 hours

Subject Code:	12AE207						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Charle’s law states that, if any gas is heated at constant pressure, its volume changes directly as its __________.

2.	Volumetric analysis of gaseous mixture is based on __________ and gravimetric analysis is based on _________.

3.	The smallest section of a nozzle is known as __________.

4.	State the classification of steam turbines based on the direction of steam flow.

5.	Write the formula to estimate the relative efficiency of a gas power cycle.

6.	State the 4 process in a Rankine cycle.

7.	State the difference between primary and secondary fuels.

8.	Define calorific value of a fuel.

9.	Define COP of a refrigeration system.

10.	What is meant by draft?

PART – B  (5 x 3 = 15 MARKS)

11.	Draw the p-v-T surface of an ideal gas and denote the equilibrium states.

12.	Enumerate the difference between the throttle and nozzle control governing of steam turbines.

13.	The efficiency of an Otto cycle is 60% and =1.5. Calculate the compression ratio of the cycle.

14.	Write the procedure to convert weight analysis of fuel to volumetric analysis.

15.	Compare vapour compression and vapour absorption refrigeration system in terms of energy used, wear and tear and performance at part loads.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	The volume of a high altitude chamber is 40 m3. It is put into operation by reducing 	pressure from 1 bar to 0.4 bar and the temperature from 25C to 5C. How many kg of air 	must be removed from the chamber during the process? Express this mass as a volume 	measured at 1 bar and 25C. Take R = 287 J/kgK for air. The values for CO2 : M=44, 	a=362850 Nm4/kg-mol, b=0.0423 m3/kg-mol, Ro = 8314 Nm/kg-mol K.			    (8)

b.	A mixture of Hydrogen (H2) and Oxygen (O2) is to be made so that the ratio of H2 to O2 is 2:1. If the temperature and pressure are 1 bar and 25C respectively, calculate the mass of O2 required and the volume of the container.										    (7)

(OR)

17.	a.	One kg of CO2 has a volume of 1 m3 at 100C. Compute the pressure by a) Vanderwaal’s 	equation b) Perfect gas equation.												    (8)

b.	4 kg of Carbon-di-oxide at 40C and 1.4 bar are mixed with 8 kg of Nitrogen at 160C and 1 bar to form a mixture at a final pressure of 0.7 bar. The process occurs adiabatically in a steady flow apparatus. Calculate the final temperature of the mixture. Take the value of Cp for CO2 as 0.5 kJ/kgK and Cp for N2 as 1.04 kJ/kgK.								    (7)

[P.T.O]

18.	Dry saturated steam at a pressure of 11 bar enters a convergent-divergent nozzle and leaves at a pressure of 2 bar. If the flow is adiabatic and frictionless, determine a) the exit velocity of steam b) ratio of cross-section at exit and that at the throat. Assume the index of adiabatic expansion to be 1.135.

(OR)

19.	Draw the stages of a pressure compounded steam turbine and explain the variation of pressure and velocity across the stages.

20.	In a constant volume Otto cycle, the pressure at the end of compression is 15 times that the start. The temperature of air at the beginning of compression is 38C and maximum temperature attained in the cycle is 1950C. Determine a) Compression ratio b) Thermal efficiency of the cycle c) Work done. Take  for air as 1.4.

(OR)

21.	Explain the principle and working of a reheat cycle and with the help of the T-S and h-S diagram show the increase in work done due to reheating.

22.	a.	A coal sample gave the following analysis by weight. Carbon 85 %, Hydrogen 6%, 	Oxygen 6% and the remainder being incombustible. Determine the minimum weight of air 	required per kg of coal for chemically correct composition.						    (5)

b.	Explain the construction of a Bomb calorimeter and its working principle in determining 	the calorific values of fuels.													  (10)

(OR)

23.	a.	An air-fuel contains 86% Carbon and 14% Hydrogen. Calculate the weight of air required 	if it is 25% in excess over that theoretically required. Determine also the higher calorific 	value if 1 kg of Carbon burning to CO2 evolves 33,800 kJ and 1 kg of Hydrogen burning to 	form H2 evolves 1,42,000 kJ.													    (7)

b.	Explain the construction and working of Orsat apparatus used for flue gas analysis with a 	neat sketch.																	    (8)

24.	Draw the line diagram of a vapour absorption refrigeration system and explain its working principle.

(OR)

25.	Explain the working of a central air-conditioning system with a neat sketch.