Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	AIRCRAFT STRUCTURES							      	     Time : 3 hours

Subject Code:	12AE212						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	A perfect frame should satisfy the relation m = ________.

2.	What do you mean by a continuous beam?

3.	Major structural member of an aircraft wing are ______and ______.

4.	Define Neutral axis.

5.	What is shear flow?

6.	Determine the Ixx of the thin walled section shown in the figure.

7.	Write the Euler’s formula for a flat plate of thickness t whose unloaded edges (length ‘a’) are free and loaded edges (length ‘b’) are simple supported.

8.	What is thin plate and how is it used for basic part of an aero plane structure?

9.	Name any four Matrix Materials.

10.	What are the limitations of composite materials?

PART – B  (5 x 3 = 15 MARKS)

11.	Explain, with example determinate and inderminate structure.

12.	With help of neat sketch explain wing construction.

13.	Define Shear center.

14.	What are the factors affect the strength of Thin walled column?

15.	What is an impact of reinforcement in composite material?

PART – C  (5 x 15 = 75 MARKS)

16.	The structure as shown in Figure is a Truss which is pinned to the floor at a point A and supported by a roller at a point D. Determine the force to all members of the Truss.

[P.T.O]

(OR)

17.	What is the Clapeyron’s theorem of three moments? Derive an expression for Clapeyron’s theorem of three moments.

18.	a.	What is spar? Explain in detail how it is used in carrying the Load?					(7)

b.	With help of neat sketch explain fuselage construction.							(8)

(OR)

19.	The fuselage section as shown in figure is subjected to a vertical shear load of 100 KN applied at a distance of 150 mm from the vertical axis of symmetry as shown, for the idealized section. Calculate the distribution of shear flow in the section.

20.	A beam having the cross section shown in figure is subjected to a bending moment of 1500 Nm in a vertical plane. Calculate the maximum direct stress due to bending and also state the point at which it acts.

(OR)

21.	A beam has the symmetrical, thin walled cross section shown in figure. Each wall of the section is flat and has the same length a and thickness t. Calculate the distance of the shear centre from the point 3.

22.	A Thin Rectangular Plate a x b is simply supported along its edges and carries a uniformly distributed load of intensity qo. Determine the deflected from of the plate and the distribution of bending moment.

(OR)

23.	a.	Write short notes on Needham Method and Gerard method.

b.	Find the compressive buckling stress for a sheet 10 cm × 10 cm × 3 cm with all four edges simply supported, assuming Et = 55 GPa . Take K =3.62.

24.	a.	Explain about the characteristic of Fiber and Matrix.

b.	What are the applications of composites in Aerospace Industry?

(OR)

25.	Write short notes on:

a.	Classification of Composite.

b.	Advantages and limitations of Composites.