Reg.  No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	AEROSPACE PROPULSION						      	     Time : 3 hours

Subject Code:	12AE218						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	__________ engine carries both Fuels and Oxidizers.

2.	Discuss the advantages of gas turbine engine over reciprocating engines?

3.	With a neat sketch explain the essential parts of a centrifugal compressor.

4.	Explain the principle of operation of a turbine.

5.	What are the requirements of a good atomizer?

6.	Define Propulsive efficiency.

7.	Write down the thrust equation for aircraft gas turbine engine.

8.	What is thrust reversal?

9.	What are the different types of nozzles?

10.	What is the storage temperature of liquid hydrogen?

PART – B  (5 x 3 = 15 MARKS)

11.	Explain the factors required for increasing the reaction rates.

12.	With a neat sketch explain inlet and exit velocity triangles for various types of blades.

13.	What are the different components of afterburner?

14.	What are the factors to be considered while designing a supersonic inlet?

15.	Compare solid propellant rocket and liquid propellant rocket.

PART – C  (5 x 15 = 75 MARKS)

16.	What are the different zones in a combustion chamber of an aircraft engine?

(OR)

17.	With a neat sketch explain the working principle of pressure feed rocket engine.

18.	Write a note on the Elementary theory for a single impulse stage turbine.

(OR)

19.	A multistage gas turbine is to be designed with impulse stages and is to operate with an inlet pressure and temperature of 6 bar and 900k and an outlet pressure of 1 bar. The isentropic efficiency of the turbine is 85%. All the stages are to have nozzle outlet angle of 75° and equal outlet and inlet blade angles. Mean blade speed of 250m/s and equal inlet and outlet gas velocities. Estimate the maximum number of stages required. Assume Cp = 1.15 kJ/kg K, γ = 1.333 and optimum blade speed ratio.

20.	Discuss the effect of shock waves in supersonic inlets.

(OR)

21.	Derive an expression for aircraft thrust and propulsive efficiency of an aircraft engine.

22.	What are the steps involved in cycle analysis of ideal turbojet engine?

(OR)

23.	Explain the working principle of afterburner with a neat sketch.

24.	What are the types of nozzle? Explain various operating conditions of a C-D nozzle with suitable sketch.

(OR)

25.	How is regressive, neutral and progressive burning of the solid propellant grown achieved? Explain with the aid of a diagram.