Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	HEAT & MASS TRANSFER							      	     Time : 3 hours

Subject Code:	12AE221						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define fin efficiency.

2.	Write down the equation for heat transfer through a composite plane wall.

3.	What is Newtonian heating or cooling process?

4.	Define Biot number.

5.	State Newton’s law of cooling.

6.	Define Grashof number.

7.	State Stefan-Boltzmann law.

8.	What is meant by reflectivity?

9.	What is ablative heat transfer?

10.	What is thrust chamber?

PART – B  (5 x 3 = 15 MARKS)

11.	State Fourier law of heat conduction.

12.	What is meant by Lumped heat analysis?

13.	Define Reynolds number and write its significance.

14.	What is meant by gray body?

15.	Write any two assumptions made while analyzing heat transfer in rocket nozzle.

PART – C  (5 x 15 = 75 MARKS)

16.	The wall of a refrigerator is made up of two mild steel plates 2.5 mm thick with a 6 cm thick glass wool in between the plates. The interior temperature is – 20oC, while the outside of the refrigerator is exposed to 40oC. Estimate the heat flow. Thermal conductivity of steel and glass wool are 23 W/mK and 0.015 W/mK respectively.

(OR)

17.	A wire of 7 mm diameter is covered with an insulating material (K = 1 W/mK). The wire temperature and ambient temperature are 80oC and 15oC. If the inside convective heat transfer co-efficient is 8.2 W/m2K, find the minimum thickness of insulation and also find the percentage of increase in the heat dissipation.

18.	An egg with mean diameter of 40 mm and initially at 20oC is placed in a boiling water pan for 4 minutes and found to be boiled to the consumer’s taste. For how long should a similar egg for same consumer be boiled when taken from a refrigerator at 5oC. Take the following properties of egg. k = 10 W/moC, ρ = 1200 kg/m3 ,Cp = 2 kJ/kgoC and heat transfer coefficient is 100 W/m2 oC. Use lump theory.

(OR)

19.	A aluminium slab of 5cm thick initially at a temperature of 400oC. It is suddenly immersed in water at 90oC. Calculate the mid plane temperature after 1 minute and also calculate the temperature inside the plate at a distance of 10 mm from the mid plane. Take heat transfer coefficient as 1800 W/m2K.

[P.T.O]

20.	Air at 20oC and at atmospheric pressure flows over a flat plate at a velocity of 3 m/s. If the plate is 1 m wide and 80oC, calculate the following at x = 300 mm from the leading edge.         a) Hydrodynamic boundary layer thickness  b) Thermal boundary layer thickness c) Local friction coefficient  d)  Average friction coefficient e) Local heat transfer coefficient f) Average heat transfer coefficient and g)  Heat transfer.

(OR)

21.	Water at 50oC enters 50 mm diameter and 4 m long tube with a velocity of 0.8 m/s. The tube wall is maintained at a constant temperature of 90oC. Determine the heat transfer coefficient and the total amount of heat transferred if exit water temperature is 70oC.

22.	Assuming sun to be black body emitting radiation at 6000 K at a mean distance of 12 x 1010 m from the earth. The diamter of the sun is 1.5 x 109 m and that of the earth is 13.2 x 106 m. Calculate the (a) total energy emitted by the sun (b) the emission received per m2 just outside the earth’s atmosphere (c) the total energy received by the earth if no radiation is blocked by the earth’s atmosphere (d) the energy received by a 2 x 2 m solar collector whosed normal is inclined at 45o to the sun. The energy loss through the atmosphere is 50% and the diffuse radiation is 20% of direct radiation.

(OR)

23.	Determine the diffusion rate of water from the bottom of a test tube of 25 mm diameter and 35 mm long into dry air at 25oC. Take diffusion co-efficient of water in air is0.28 x l0-4m2 /s.

24.	What is aerodynamic heating? Explain the cooling methods to reduce aerodynamic heating.

(OR)

25.	Explain the different methods of cooling thrust chambers.