Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	FINITE ELEMENT ANALYSIS						      	       Time: 3 hours

Subject Code:	12AE222						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	State different types of boundary conditions.

2.	What are the methods generally associated with finite element analysis?

3.	What is meant by discretization?

4.	Mention any two properties of stiffness matrix.

5.	What is CST element?

6.	Define plane stress analysis.

7.	Define shape function.

8.	What is meant my degrees of freedom?

9.	Write different modes of heat transfer.

10.	Write down the stiffness matrix equation for one dimensional heat conduction element.

PART – B  (5 x 3 = 15 MARKS)

11.	During discretization, mention the places where it is necessary to place a node.

12.	Define Super-Parametric, Sub-Parametric and Iso-Parametric elements.

13.	State the assumptions made while finding the forces in a truss.

14.	State the assumptions in the theory of pure torsion.

15.	Write the advantages and disadvantages of Boundary element method.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the step by step procedure of finite element method with suitable example.

(OR)

17.	Solve the following equations using Gauss – Elimination method.

3x + y – z = 3

2x – 8y + z = -5

x -  2y + 9z = 8

18.	The following differential equation is available for a physical phenomenon:

d2ydx2−10 x2=5; 0 ≤ x ≤ 1.

The boundary conditions are: y(0) = 0 and y(1)=0;  by using Galerkin’s method  of weighted residual method, find an approximate solution of the above differential equation and also compare with exact solution.

(OR)

[P.T.O]

19.	Consider a bar as shown in figure. An axial load of 200 kN is applied at point P. Take A1 = 2400 mm2, E1 = 70 X 109 N/m2, A2 = 600 mm2, E2 = 200 X 109 N/m2. Calculate the following

a.	The nodal displacement at point P.		b.	Stress in each material.

c.	Reaction force.

20.	 Consider a three bar as shown in figure. It is given that E = 2 X 105 N/mm2. Calculate

a.	Nodal Displacements		b.	Stress in each member	c.	Reactions at the support.

Take Area of first element, second element and third element as 2000mm2, 2500mm2 and 	2500mm2 respectively.

(OR)

21.	Determine the Eigen values and frequencies for the stepped bar shown in figure.

Take young’s modulus, E = 2 X 105 N/mm2

Unit weight density, ρ = 0.8 X 10 – 4 N/mm2

22.	For the plane stress element shown in figure, the nodal displacements are

U1 = 2.0 mm				v1 = 1.0 mm

U2 = 0.5 mm				v2 = 0.0 mm

U3 = 3.0 mm				v3 = 1.0 mm

Determine the element stresses σx,σy, τxy, σ1 and σ2 and the principal angle θP. Let E= 210 GPa, ν = 0.25 and t = 10 mm. All co-ordinates are in millimeters.

(OR)

23.	Evaluate the Cartesian co-ordinate of the point P which has local co-ordinates ε = 0.6 and η = 0.8 as shown if figure.

24.	A wall of 0.6 m thickness having a thermal conductivity of 1.2 W/mK. The wall is to be insulated with a material of thickness 0.06 m having an average thermal conductivity of 0.3 W/mK. The inner surface temperature is 10000C and outside of the insulation is exposed to atmospheric air at 300C with heat transfer coefficient of 35 W/m2K. Calculate the nodal Temperatures.

(OR)

25.	For the two dimensional sandy soil region shown in figure, determine the potential distribution and velocity gradient. The potential (fluid head) on the left side is a constant 10.0 m, and that on the right side is 0.0 m. The upper and lower edges are impermeable. The permeabilities are kx = ky = 25 X 10 – 5 m/s. Assume unit thickness.