Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	FLUID MECHANICS								      	      Time: 3 hours

Subject Code:	12BT204						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The study of fluid at rest is called ________.

2.	Water is an example for ________ fluid.

3.	The working of orifice meter is based on ________ theorem.

4.	Globe valve is an example for ________ valve.

5.	Major energy loss in pipe flow is due to ________.

6.	What are the two types of fluidization?

7.	Mixing of two liquids is called ________.

8.	Equipment used for mixing plastic or deformable solids is called ________.

9.	Reciprocating pump is a ________ pump.

10.	The pump which uses the centrifugal force to increase the velocity is called ________.

PART – B  (5 x 3 = 15 MARKS)

11.	What are monometers? Classify them and give example.

12.	A pipe tapers from 75mm dia to 45mm dia. The quantity of water flowing through the pipe is 0.15m3/sec. Find the velocities at the two sections.

13.	What is minimum bubbling velocity?

14.	What is mixing index?

15.	What is the use of foot valve in centrifugal pump?

PART – C  (5 x 15 = 75 MARKS)

16.	An U tube differential manometer connects two pipes. Pipe A contains carbon tetrachloride having specific gravity of 1.594 under a pressure of 118 KN/m2. The pipe B contains oil of specific gravity 0.8 under a pressure of 200KN/m2. The pipe A lies 2.5m above pipe B. The center of pipe B is at the level of free surface of mercury. Find the difference in level of mercury.

(OR)

17.	State and Prove Bernoulli theorem with the assumptions made.

18.	Derive the expression for discharge co-efficient for venturimeter.

(OR)

19.	What is the use of valves? Explain various types of valves.

20.	What is fluidization? Explain its type in detail.

(OR)

21.	Derive the Ergun equation, starting from flow through packed beds.

22.	What are the criteria for mixer effectiveness and what is the significance of mixing index?

(OR)

23.	What are the equipments used for mixing of pastes? Explain in detail.

24.	How pumps are classified? Explain the working principle of reciprocating pump.

(OR)

25.	Explain the principle and operation of fans, compressors and blowers.