Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	BIOPROCESS CALCULATIONS					      	     Time : 3 hours

Subject Code:	12BT219						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	1 inch = ___________ m.

a.	25.4              	b.	0.0254           	c.	0.3048            	d.	0.01

2.	Molecular weight of H2SO4 = ___________.

a.	98                  b.	49                       	c.	40                        	d.	54.75

3.	___________ process, weak liquor is converted into thick liquor.

a.	Distillation    b.	Evaporation       	c.	Mixing          	d.	Filtration

4.	Overall material balance for distillation process is ___________.

5.	___________ = (moles of A reacted/moles of A charged ) x 100.

a.	% conversion of A    	b.	% excess of A   	c.	% loss of A

6.	Stoichiometric ratio is defined as ___________.

7.	Empirical equation for heat capacity is ___________.

8.	Heat capacity unit is ___________.

a.	kJ/K             	b.	kJ/kmol.K            	c. 	kJ/kmol             	d. 	kJ

9.	Heat of reaction is ___________.

10.	Heat of formation unit is ___________.

a.	kJ/K            	b.	kJ/kmol.K           	c.	kJ/kmol               	d. 	kJ

PART – B  (5 x 3 = 15 MARKS)

11.	What is the average molecular weight of air and its composition by weight?

12.	It is desired to make up 1,000kg of a solution containing 35% by weight of a substance “A”. Two solutions are available, one containing 10 weight % of “A” and other containing 50 weight % of “A”. How many kilograms of each solution will be required?

13.	Define: Material balance.

14.	A stream of nitrogen flowing at a rate of 100 kmol/hr is heated from303K to 373K. Calculate the heat that must be transferred Data:

Cp° for nitrogen = 29.5909 -5.141x10-3T+11.1829x10-6T2 - 4.968x10-9T3

15.	Define: Hess’s law.

PART – C  (5 x 15 = 75 MARKS)

16.	A solution containing 55% benzene, 28% toluene and 17% xylene by weight is in contact with its vapour at 373K. Calculate the total pressure and molar composition of the liquid and vapour.

Data: vapour pressure data at 373K, Benzene=178.60kPa, Toluene=74.60kPa & Xylene=28kPa.

(OR)

17.	Find the equivalent weight of a) HCl b) NaOH  c) Na2CO3  and d) H2SO4

[P.T.O]

18.	Dryer system handles 10,000kg/day of wet solids. Wet solids containing 50% solids and 50% water are fed to the first dryer. From the first dryer the product that comes out has 20% moisture. This is admitted to the second dryer from which the product coming out has 2% moisture. Calculate the percentage of original water that is removed in each dryer and final weight of the product.

(OR)

19.	In production of chlorine gas by oxidation of hydrochloric acid gas, air is used 30% in excess of that theoretically required. Based on 4 kmol HCl, calculate: a) the weight ratio of air to hydrochloric acid gas in feed. b) if oxidation is 80% complete, find the composition of product stream on mole basis.

20.	A feed containing 60 mole% A, 30 mole% B and 10 mole % inerts enters a reactor. 80 % of original A reacts according to the following a reaction

2A+B→C

Find the composition of the product stream on mole basis, based on the given composition.

(OR)

21.	Nitrogen is reacted with hydrogen to produce Ammonia. From the reaction; calculate a) The molar flow rate of hydrogen corresponding to nitrogen feed rate of 25 kmol/hr, if they are fed in the stoichiometric proportion and b) The kg of ammonia produced per hour if percent conversion is 25 and nitrogen feed rate is 25 kmol/hr.

22.	Flue gas leaving the boiler stack at 523K have the following composition:

CO2=11.31%, H2O=13.04%, O2=2.17% and N2=73.48% (by volume). Calculate the heat lost in

1 kmol of gas mixture above 298K, using the heat capacity data given below:

Cp = a+bT+cT2+dT3, kJ/(kmol.K)

| Gas | a | b x103 | c x106 | d x109 |
| --- | --- | --- | --- | --- |
| CO2 | 21.3655 | 64.2841 | - 41.0506 | 9.7999 |
| H2O | 32.4921 | 0.0796 | 13.2107 | - 4.5474 |
| O2 | 26.0257 | 11.7551 | - 2.3426 | - 0.5623 |
| N2 | 29.5909 | - 5.141 | 13.1829 | - 4.968 |

(OR)

23.	Toluene is to be heated from 290K to 350K at the rate of 250gm/sec. Calculate the heat to be supplied to toluene using the heat capacity data given below

Cp = a+bT+cT2+dT3, kJ/(kmol.K)

| Gas | a | b x103 | c x106 | d x109 |
| --- | --- | --- | --- | --- |
| Toluene | 1.8083 | 812.223 | -151.67 | 1630.01 |

24.	a.	When liquid benzene is completely burned to carbon dioxide and liquid water, the standard 	heat of combustion is – 3267.6 kJ/mol. The standard heat of combustion of hydrogen to 	liquid water is – 285.83 kJ/mol and that of carbon to carbon dioxide gas is – 393.51 	kJ/mol. Calculate the standard heat of formation of liquid benzene.					  (10)

b.	Calculate the heat that must be removed in cooling 32 kg of oxygen from 488K to 313K 	using   Cp° data:    Cp° =  a+bT+cT2+dT3, (kJ/kmol.K)

| Gas | a | b x 103 | c x 106 | d x 109 |
| --- | --- | --- | --- | --- |
| Oxygen | 26.0257 | 11.7551 | - 2.3426 | - 0.5623 |

(5)

(OR)

25.	Obtain an empirical expression relating the heat of reaction and the temperature of the reaction for the following reaction.

SO2 + ½ O2                 SO3

Using the same expression, calculate the heat of reaction at 773K

Cp° = a+bT+cT2+dT3, kJ/(kmol.K)

Data:

| Component | ΔHf o kJ/kmol |
| --- | --- |
| SO3 | -3,95,720 |
| SO2 | - 2,96,810 |

| Component | a | b x103 | c x106 | d x109 |
| --- | --- | --- | --- | --- |
| SO3 | 22.036 | 121.624 | - 91.867 | 24.369 |
| SO2 | 24.771 | 62.948 | - 44.258 | 11.122 |
| O2 | 26.026 | 11.755 | - 2.343 | - 0.562 |