Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	BASIC PROCESS EQUIPMENT DESIGN			      	     Time : 3 hours

Subject Code:	12BT241								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	The direction of process flow is represented by the symbol as _________.

2. 	The symbol for gate valve is _________.

3. 	Vena contracta formed during flow of liquid through an orifice meter has _________ liquid cross sectional area.

4. 	The valve commonly used in pipes larger than 2 inch in diameter is _________.

5. 	In a single effect evaporator, the economy is _________.

6. 	Double pipe heat exchangers are used when heat transfer area required is _________.

7. 	In batch distillation with constant reflux, overhead product composition _________ with time.

8. 	Milk is dried usually in a _________ dryer.

9. 	In a CSTR _________ varies with time.

10. 	A back mix reactor is same as _________.

PART – B  (5 x 3 = 15 MARKS)

11. 	Discuss the application of design engineering.

12. 	What is the difference between Venturimeter and Rotameter from the point of view of the flow measurement?

13. 	What is Duhuring’s line and Boiling point rise?

14. 	How column diameter is calculated for bubble cap tray tower?

15. 	Give the performance equation for batch reactor, mixed flow reactor and plug flow reactor.

PART – C  (5 x 15 = 75 MARKS)

16. 	Explain the plant layout for the citric acid production.

(OR)

17. 	Describe the plant layout for the production of penicillin.

18. 	A properly designed venturimeter has a throat diameter of 30 cm and inlet openings diameter of 60 cm.  If it is inserted in a pipe line of 60 cm through which dry chlorine gas is flowing, the reading of the venturimeter connected to a U tube mercury manometer is 1.3 cm of Hg.  Calculate the rate of flow of chlorine gas in kg/hr.  Assume the density of mercury to be 13.6 gm/cc and that of chlorine gas at the conditions involved as 0.009 gm/cc.  The coefficient of discharge can be taken as 0.98.

(OR)

19. 	A sharp-edged orifice connected to a manometer is used for measuring the flow rate of brine of specific gravity flowing through a 7.5 cm inner diameter pipe.  The maximum flow rate is not to exceed 400 mm Hg.  Calculate the size of the orifice.

[P.T.O]

20. 	Two liquids are flowing in a double pipe heat exchanger both at the rate of 1770 kg/hr.  The inner pipe of the heat exchanger has inner diameter of 2.5 and wall thickness of 0.125 cm.  The inner pipe is made up copper having thermal conductivity K=328 kcal/hr, m2 oC/m.  The outer pipe of heat exchanger has an internal diameter of 5.0 cm.  For simplicity assume that both liquids in the inner tube is being cooled from 70oC to 55oC at the expense of the liquid flowing in the outer tube which is entering at 20oC.  Calculate the length of the heat exchanger required if the liquid are flowing in a counter-current manner.  Physical properties of water: Viscosity at 62.5oC= 0.452 cp, Viscosity at 27.5oC=0.840, Thermal conductivity at 62.5oC=0.57 kcal/hrm2oC/m, at 27.5oC=0.528 kcal/hr m2 oC/m.

(OR)

21. 	A single effect evaporator operates at 114 mm Hg absolute pressure to concentrate 4500 Kg/hr of a 8% solution of a caustic soda to 36%. Steam is available at 120oC and the overall heat transfer coefficient is 1500 kcal/hr m2 oC . The heating surface in the evaporator is 132 cm below the liquid level. Using following data and equation, design suitable evaporator:

B.P.R of solution  = 28oC

Feed temperature =20oC

Specific heat of feed  =0.96 kcal/kg oC

Specific heat of  product =0.83 kcal/kg oC

Specific gravity of boiling liquid =1.4

B.P.R due to hydrostatic head is given by

∆th = 0.827 (TR VR/λs) ∆p

Evaporator tubes are 37.5 mm O.D (34.2 mm inner diameter) and of height 150 cm arranged on 47.65 mm square pitch. Estimate steam consumption and given essential dimensions of the evaporator.

22. 	A feed mixture containing 44 mole % benzene and remaining toluene is fed to a continuous fractionating column at the rate of 9 t/hr to yield an overhead product of 97.4 mole % benzene and a bottom product of 2.3 mole % benzene.  A reflux ratio of 3.5 is to be used.  The molar latent heat of benzene and toluene may be taken as 30 MJ/kmol.  The feed is a liquid at 295 K and has boiling point of 370.4 K.  Design a suitable fractionating column and compute the following.

a.	The number of theoretical plates and position of feed plate.

b.	The quantity of steam at 240 kN/m2 required in the reboiler.

c.	The column height for a plate spacing of 450 mm.

d.	The column diameter if it operates at atmospheric pressure and vapor velocity of 60 	cm/sec.

Data: Vapor flow in enriching section is 2.48 mole/mole of feed, boiling point of benzene is 353.3 K, boiling point of toluene is 383.6 K and specific heat of feed is 1.84 kJ/kg K.  The equilibrium data for benzene-toluene system is:

| x | 0.1 | 0.2 | 0.3 | 0.4 | 0.5 | 0.6 | 0.7 | 0.8 | 0.9 | 1.0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| y | 0.22 | 0.38 | 0.51 | 0.63 | 0.7 | 0.78 | 0.85 | 0.9 | 0.96 | 1.0 |

(OR)

23. 	A continuous fractionating column is to be designed for the separation of a mixture containing 0.5 mole fraction of n-heptane and the rest is n-octane.  If the overhead and bottom products are to have 99% purity and the column is to operate at atmospheric pressure with reflux ratio of 2.5.  Find out the number of theoretical plates required.  The feed is admitted as a saturated liquid to the column at a rate of 9t/hr.  For the following conditions find the diameter of the column.  Top column temperature is 371 K, bottom column temperature is 398 K, vapor velocity in the column is 0.5 m and plate spacing is 0.45m. The vapor-liquid equilibrium data for heptane-octane system is

| x | 0.13 | 0.22 | 0.32 | 0.46 | 0.57 | 0.69 | 0.82 | 0.92 | 1.00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| y | 0.24 | 0.37 | 0.50 | 0.65 | 0.74 | 0.83 | 0.91 | 0.96 | 1.00 |

24. 	Pyrolysis of methylacetoxy propionate at temperature near 500ºC has been studied.

CH3COOCH(CH3) COOCH3            CH3COOH + CH2  = CHCOOCH3

Below 565ºC, the pyrolysis reaction is essentially first order with a rate constant given by

K = 7.8 x 109 e -19220/T sec -1

where T is in Kelvin. It is desired to design a plug flow reactor to operate isothermally at 500ºC. What length of 16 cm pipe will be required to convert 90% of the raw feed stock to methyl acrylate? The feed stock enters at 5atm at a flow rate of 230 kg/hr. Find the mean residence time of the material in PFR.

(OR)

25. Following the gas phase reaction takes place at 500ºC.

4PH3 (g)              P4 (g) + 6 H2 (g) with rPH3 = 85 h-1

Find the volume of the plug flow reactor operating at this temperature and 5.5atm giving 80% conversion for a feed of 2.5 kmoles / PH3 per hour.