Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	BASIC CIVIL ENGINEERING						      	     Time : 3 hours

Subject Code:	12CE101						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Write the branches of Civil Engineering.

2.	Mention the role of Civil Engineers in natural resources development and conservation.

3.	How are loads classified with respect to area on which they are distributed?

4.	Define Surveying.

5.	List out major constituents of cement.

6.	What is meant by M15 concrete?

7.	How are sleepers classified on the basis of materials?

8.	Recall the functions of a Hanger.

9.	What are the basic sources of water supply?

10.	Explain the term “sludge”.

PART – B  (5 x 3 = 15 MARKS)

11.	Summarize the function of a civil engineer.

12.	Explain the principles of surveying.

13.	Draw stress-strain diagram for mild steel.

14.	Define Gauge and classify it.

15.	Differentiate slow sand and rapid sand filters.

PART – C  (5 x 15 = 75 MARKS)

16.	Write short on the scope of Civil engineering.

(OR)

17.	Explain the role of civil engineer in Infrastructural development.

18.	A series of offsets were taken from a chain line to a curved boundary line at regular interval of 10m.The length of the offsets are 2.2m, 1.4m, 2.8m, 2.2m, 2.6m, 4m, 3.2m, 3.4m and 3.8m. Find the area of the strip between chain line and boundary line by all the available methods.

(OR)

19.	A steel rod of 20mm diameter and 1m length is subjected to an axial pull of 50kN. Determine the stress, strain and elongation of the rod taking young’s modulus, E=2x105N/mm2.

20.	Explain in detail the requirements of a good building stone.

(OR)

21.	List out the characteristics of the following building materials a) Bricks  b) Timber.

22.	Formulate the general layout of an Airport and explain its components.

(OR)

23.	Enumerate the classification of Highway.

24.	Briefly describe the points to be considered in the selection of a site for a dam and its classification.

(OR)

25.	Elaborate the steps involved in water purification process.