Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	SANITARY ENGINEERING						      	     Time : 3 hours

Subject Code:	12CE226						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Enumerate the types of sewerage systems.

2.	Compare the aerobic and anaerobic decomposition of sulphurous organic matter.

3.	List down the different types of sewers.

4.	What are the different materials adopted for sewers?

5.	What are skimming tanks?

6.	What is meant by suspended growth process?

7.	Write the steps involved in the process of sludge digestion.

8.	What is bioremediation?

9.	Write any one good quality of a trap.

10.	What is the minimum amount of dissolved oxygen that should available in the river for effective survival of fish?

PART – B  (5 x 3 = 15 MARKS)

11.	Define relative stability and population equivalent.

12.	Explain the testing of sewers.

13.	State the advantages of oxidation pond.

14.	What do you understand by anaerobic digestion? Give the practical application of it.

15.	What is meant by oxygen sag curve?

PART – C  (5 x 15 = 75 MARKS)

16.	A combined sewer system is to be designed for a city having an area of 60 hectares and a population of 20,000. If the average coefficient of runoff for the area is 0.6 and the time of concentration is 30 minutes, calculate the discharge for the sewer system. Assume per capita water demand as 135lpcd.

(OR)

17.	Explain in detail the characteristics of sewage.

18.	Determine the size of a circular sewer for a discharge of 800 l/s running 0.6 full. Assume          s = 0.0001 and n = 0.015.

(OR)

19.	With neat sketch explain Manholes, Drop Manholes, Automatic Flushing tanks, Lamp Holes, Inverted Siphon.

20.	a.	Write down the different objectives of sewage treatment.							    (6)

b.	With neat sketch explain the working of a grit chamber. Mention the design features.   (9)

(OR)

[P.T.O]

21.	a.	Explain the functioning of a stabilization pond.									    (7)

b.	Sewage flows from a primary settling tank to a standard rate trickling filter at the rate of 5 	MLD having a 5 day BOD of 150 mg/l. Determine the volume and depth of the filter. 	Adopt a surface loading of 2500 l/m2/day and organic loading of 165 g/m3/day. What is the 	efficiency of the filter according to NRC formula?								    (8)

22.	a.	What are the stages of sludge digestion? Explain each in detail with neat sketch.		    (8)

b.	What is meant by sewage sickness?												    (7)

(OR)

23.	Write a detailed note on self purification of stream and the factors affecting self purification of streams.

24.	a.	Explain one pipe and two pipe system with neat sketch.							    (7)

b.	With neat sketch explain the working of a septic tank.								    (8)

(OR)

25.	a.	Draw a neat layout of house drainage works.										    (7)

b.	Explain the different water conservation measures and the necessity of it.			    (8)