Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	RAILWAYS, AIRPORTS AND HARBOURS			      	      Time: 3 hours

Subject Code:	12CE228						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What do you mean by Gauge?

2.	What are the materials used for sleepers in Indian Railways?

3.	State the device used to regulate the movement of trains.

4.	Give the two types of Rail joints.

5.	What do you mean by turning zones in airport?

6.	Give the wind direction indicator used in airport.

7.	State the structure used to break the waters and dissipate the energy at harbor.

8.	State the system which is used to prevent damage to both the vessel and berth during the berthing process.

9.	Name the two operators which influence the genetic algorithms performance.

10.	Which AI method possesses a large number of processing elements called nodes/neurons which operate in parallel?

PART – B  (5 x 3 = 15 MARKS)

11.	Draw a typical cross section of a permanent way and indicate its components.

12.	Detail the simple layout of a turnout with neat sketch.

13.	Brief about holding apron.

14.	Classify the different types of harbor.

15.	Brief the three main stages in Fuzzy Logic.

PART – C  (5 x 15 = 75 MARKS)

16.	Discuss the curves on Railways with neat sketches.

(OR)

17.	a.	Discuss the need for widening of gauges on curves. 							            (7)

b.	Wheel base of a moving vehicle is 4.12m.  Degree of curve is 50 and flanges project 3.2cm 	below the top of rail. Determine extra width required on the curve. 					    (8)

18.	Explain with neat sketch the location of various signals at a railway station.

(OR)

19.	Explain with neat sketches, the layout of various stations indicating various passenger facilities.

20.	How are various provisions of parking of vehicles made at an airport? Explain any four pattern of layouts with neat sketches.

(OR)

21.	Lengths of a runway at mean sea level, standard temperature and zero gradients in 1600m.  The site has an elevation of 320m, with a reference temperature 33.60C.  The runway has to be constructed with an effective gradient of 0.25%. Determine the actual length of the runway at the site.

[P.T.O]

22.	Enumerate various marine surveys involved in locating Harbour.

(OR)

23.	Discuss any four coastal structures with neat sketches and state their location and functions.

24.	Discuss about the Intelligent Transport System.

(OR)

25.	Explain the applications of Artificial Intelligence in Transportation.