Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	APPLIED CHEMISTRY							      	     Time : 3 hours

Subject Code:	12CH201						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	What is the hardness of the softened water obtained from Zeolite process?

2. 	CaCl2 is a ___________ hardness producing salt.

3. 	Name the moulding method used for thermosetting plastics.

4. 	The polymer prepared from vinyl chloride is ___________.

5. 	Write any two units of calorific value.

6. 	The calorific value of LPG is ___________.

7. 	What is the electrode potential of standard hydrogen electrode?

8. 	Give two examples for corrosion inhibitor.

9. 	Which reagent is used to reduce the friction between two moving parts?

10. 	Give one example for acid refractory material.

PART – B  (5 x 3 = 15 MARKS)

11. 	Differentiate between temporary and permanent hardness.

12. 	Define condensation polymerization. Give one example.

13. 	How will you improve the anti knocking characteristics of a fuel?

14. 	Write note on galvanic corrosion.

15. 	How is calcium based grease prepared? Write its properties.

PART – C  (5 x 15 = 75 MARKS)

16. 	a. 	Define reverse osmosis. How is sea water desalinated by reverse osmosis?			    (8)    	b. 	Write note on calgon conditioning and break point chlorination.					    (7)

(OR)

17. 	a.	Describe Zeolite process of softening the hard water with a neat diagram.  		          (10)	 b. 	Calculate the permanent, temporary and total hardness of water sample containing

Ca (HCO3)2=80mg/l;    MgCl2 =40 mg/I;   CaSO4 = 60 mg/l.   						    (5)

18. 	a.	Explain the fabrication of plastics by compression moulding.						    (7)

b.	Write the preparation and uses of polyethylene and bakelite.						    (8)

(OR)

19. 	a.	List out the drawbacks of raw rubber. How will you prepare vulcanized rubber?  	    (8)

b.	Differentiate between thermoplastics and thermosetting plastics.  					    (7)

20. 	a. 	How can you analyze the constituents of coal by proximate analysis? Write its 	significances. 																  (10)

b.	Calculate the gross and net calorific value of coal containing carbon -  85%; hydrogen – 	8% ; Oxygen-0%;    Sulphur – 1% latent heat of steam =587cal/g.					   (5)

(OR)

[P.T.O]

21. 	a.	Explain the manufacture of coke by Otto – Hoffman method with a neat diagram.  	  (10)

b.	Write note on solid rocket propellants. 											    (5)

22. 	a.	Derive Nernst equation for electrode potential.									    (7)

b.	Write note on  i.	Concentration cell corrosion 	ii.	Sacrificial anodic protection.       (8) (OR)

23. 	a.	Explain the construction and working of H2 – O2  fuel cell. 						  (10)

b. 	Explain the mechanism of rusting of iron.					 					    (5)

24. 	a.	Give an account of the steps involved in the manufacture of refractories.			  (10)

b.	Write note on graphite.  														    (5)

(OR)

25. 	Write note on the following:

a.	Preparation and uses of silicon carbide. 											    (5)

b. 	Any four electrical insulators. 													    (5) c.	Functions of lubricants.														    (5)