Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	ELECTRON DEVICES								      	     Time : 3 hours

Subject Code:	12EC201										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is the η value for germanium and silicon?

2.	What are the classifications of semiconductor?

3.	Define emitter injection efficiency.

4.	Write Ebers Moll equation.

5.	Draw the hybrid model for a two port network.

6.	Write down the applications of CE amplifier.

7.	Give the symbol of DIAC.

8.	What is SCR?

9.	What is the other name of tunnel diode?

10.	Give the symbol of photo diode.

PART – B  (5 x 3 = 15 MARKS)

11.	Draw the energy band structure of a semiconductor.

12.	Draw the H parameter circuit for a CB transistor.

13.	Compare FET and MOSFET.

14.	What is mean by pinch off region?

15.	Mention the applications of Tunnel diode.

PART – C  (5 x 15 = 75 MARKS)

16. 	a.	With necessary diagrams, explain the energy band description of conductors, 	semiconductors and insulators.													  (10)

b.	Write short notes on drift and diffusion currents. 									    (5)

(OR)

17. 	What is a PN junction diode? Explain the structure, operation and characteristics of PN junction diode.

18. 	What are the various transistor parameters and current components of a transistor? Explain in detail.

(OR)

19. a.	Describe the static characteristics of a NPN transistor in CE configuration. 			  (10)

b.	Give the comparison between CC, CB and CE.									    (5)

20. 	Draw the small signal model for CC amplifier and derive voltage gain, current gain, input impedance and output impedance.

(OR)

21. 	What are hybrid parameters? Draw the hybrid model for a two port network. How will you obtain h-parameters from the characteristics of CE configuration? What are the limitations of h parameters?

[P.T.O]

22. 	With neat diagrams, explain the structure, operation and characteristics of Enhancement mode and depletion mode MOSFET.

(OR)

23. 	Explain the construction, operation, volt ampere characteristics, and application of SCR.

24. 	With a neat diagram explain the working of: a) Tunnel diode	and b) Photodiode

(OR)

25.	Explain the construction, operation, VI characteristics, and applications of TRIAC.